ECE R13H BRAKING (M1) 2009/11/11 00-S7、2010 * 00-S8、2010/01/07 00-R/C1、2010/01/07 00-S7/C1 小客車煞車

| 增/修內容 | 原內容 | 修訂國內法規條文草案 | 對應國內法規條文 |
| --- | --- | --- | --- |
| ECE R13H 00-S7/C1 5.2.24.1. As an alternative to the requirement of paragraph 5.2.24., vehicles of categories M1 and N1 with a mass in running order > 1,735 kg may be equipped with a vehicle stability function which includes roll-over control and directional control and meets the technical requirements of Annex 21 to Regulation No. 13. ECE R13 11 5.2.1.32. Subject to the provisions of paragraph 12.4. to this Regulation, all vehicles of categories M2, M3, N2 and N3 */ having no more than 3 axles shall be equipped with a vehicle stability function. This shall include roll-over control and directional control and meet the technical requirements of Annex 21 to this Regulation. ECE R13 11 5.2.2.23. Subject to the provisions of paragraph 12.4. to this Regulation, all vehicles of categories O3 and O4 22/ having no more than 3 axles and equipped with air suspension shall be equipped with a vehicle stability function. This shall include at least roll-over control and meet the technical requirements of Annex 21 to this Regulation. ECE R13H 00-S7/C1 2.24.*/ "Ackerman steer angle" means the angle whose tangent is the wheelbase divided by the radius of the turn at a very low speed. 2.25. "Electronic Stability Control System" or "ESC System" means a system that has all of the following attributes: 2.25.1. That improves vehicle directional stability by at least having the ability to automatically control individually the braking torques of the left and right wheels on each axle 1/ to induce a correcting yaw moment based on the evaluation of actual vehicle behaviour in comparison with a determination of vehicle behaviour demanded by the driver; 1/An axle group shall be treated as a single axle and dual wheels shall be treated as a single wheel. 2.25.2. That is computer controlled with the computer using a closed-loop algorithm to limit vehicle oversteer and to limit vehicle understeer based on the evaluation of actual vehicle behaviour in comparison with a determination of vehicle behaviour demanded by the driver; 2.25.3. That has a means to determine directly the value of the vehicle's yaw rate and to estimate its side-slip or side-slip derivative with respect to time; 2.25.4. That has a means to monitor driver steering inputs; and 2.25.5. That has an algorithm to determine the need, and a means to modify propulsion torque, as necessary, to assist the driver in maintaining control of the vehicle. 2.26. "Lateral acceleration" means the component of the acceleration vector of a point in the vehicle perpendicular to the vehicle x axis (longitudinal) and parallel to the road plane. 2.27. "Oversteer" means a condition in which the vehicle's yaw rate is greater than the yaw rate that would occur at the vehicle's speed as a result of the Ackerman steer angle. 2.28. "Side-slip or side-slip angle" means the arctangent of the ratio of the lateral velocity to the longitudinal velocity of the centre of gravity of the vehicle. 2.29. "Understeer" means a condition in which the vehicle's yaw rate is less than the yaw rate that would occur at the vehicle's speed as a result of the Ackerman steer angle. |  | 四十二之三、動態煞車： 1. 實施時間及適用範圍： 1.1 中華民國○年○月○日起，新型式之Ｍ1、Ｎ1、Ｏ1、O2、L1及L3類車輛，其動態煞車，應符合本項規定。 1.1.1 中華民國○年○月○日起，各型式之M1及選擇符合5.規範之N1類車輛，應配備符合5.5規定之車輛穩定性電子式控制系統（ESC）。 1.1.2 車重(running order)大於一七三五公斤之M1及N1類車輛，新型式自中華民國○年○月○日起，各型式自中華民國○年○月○日起，如具備包含翻覆性控制及方向性控制之車輛穩定性電子式控制功能（VSF）而未配備ESC者，則應符合6.9車輛穩定性電子式控制功能（VSF）之特殊規定。 1.2 中華民國○年○月○日起，新型式之M2、M3、N2、N3及O3、O4類車輛，其動態煞車，應符合本項規定。 1.3 下述車輛，新型式自中華民國○年○月○日起，各型式自中華民國○年○月○日起，應具有符合6.9規定之車輛穩定性電子式控制功能（VSF）： 1.3.1 M2、M3、N2及N3類車輛且其輪軸組未愈三軸者。 1.3.2 O3及O4類車輛且其輪軸組未愈三軸並配備氣壓式懸吊者。 1.4 本項不適用於： 1.4.1 設計車速不大於二五公里/小時之車輛。 1.4.2 無法與設計車速大於二五公里/小時曳引車聯結之拖車。 1.5 同一申請者同一年度同型式規格之M1或L3類車輛，申請少量車型安全審驗且總數未逾三輛者，得免符合本項「動態煞車」規定。 1.6 同一申請者同一年度同型式規格車輛，申請逐車少量車型安全審驗且總數未逾二十輛者，得免符合本項「動態煞車」規定。 1.7 同一申請者同一年度同型式規格之M1或N1類車輛申請少量車型安全審驗且總數未逾二十輛或機關、學校進口自行使用之N2、N3類或丙、丁類大客車，得免符合本項「動態煞車」規定中第二煞車系統性能、能量儲存裝置測試及5.2.3.2.3或6.2.3.4.3之電池充能狀態確認。 2. 名詞釋義 … 2.2.27 車輪鎖定（Wheel lock）：指發生打滑率達一．００之狀態。 2.10 緊急煞車作動信號(Emergency Braking Signal)：如5.1.13及6.1.21所述用以表示緊急煞車之邏輯信號。 2.11 阿克曼轉向角： 係指在速度非常低之下的tangent θ等於軸距除以轉彎半徑。 2.12車輛穩定性電子式控制系統（ESC）或ESC系統：係指一擁有下述所有特性之系統： 2.12.1 在各軸(軸組於測試時視為單軸，複輪則視為單輪)上以至少能個別自動控制左右輪煞車扭矩之方式產生正確偏移力矩，提昇車輛於行駛方向之穩定度，該偏移力矩係估算車輛實際狀態與依駕駛人要求而應對應產生狀態之比較。 2.12.2 使用閉迴路演算法之控制電腦以限制車輛轉向過度或不足，該轉向控制係估算車輛實際狀態與依駕駛人要求而應對應產生狀態之比較。 2.12.3 具有方法可直接決定車輛橫擺角速度（Yaw rate）數值且能估計其側滑或由時間所導出之側滑率。 2.12.4 具有方法可監控駕駛轉向輸入之訊號；且 2.12.5 具有決定需求之演算法，且擁有必要時可調整推進扭矩之方法，以幫助駕駛控制車輛。 2.13 側向加速：係指加速度向量於垂直車輛X軸(縱向)且平行於路面之分量。 2.14轉向過度：係指一情況，其車輛橫擺角速度（Yaw rate）大於車輛由阿克曼轉向角所導出該速度可能產生之橫擺角速度（Yaw rate）。 2.15側滑或側滑角：係指於車輛重心處橫向速度與縱向速度之正弦值。 2.16 轉向不足：係指一情況，其車輛橫擺角速度（Yaw rate）低於車輛由阿克曼轉向角所導出該速度可能產生之橫擺角速度（Yaw rate）。 | 四十二之二、動態煞車：自一０二年一月一日起實施 1. 實施時間及適用範圍： 1.1 中華民國一０二年一月一日起，新型式之Ｍ、Ｎ、Ｏ、L1及L3類車輛及中華民國一０四年一月一日起，各型式之L1及L3類車輛，其動態煞車，應符合本項規定。符合本基準項次「四十二之一」規定之Ｍ、Ｎ及Ｏ類車輛，亦視同符合本項規定。 1.2 本項不適用於： 1.2.1 設計車速不大於二五公里/小時之車輛。 1.2.2 無法與設計車速大於二五公里/小時曳引車聯結之拖車。 1.3 同一申請者同一年度同型式規格之M1或L3類車輛，申請少量車型安全審驗且總數未逾三輛者，得免符合本項「動態煞車」規定。 1.4 同一申請者同一年度同型式規格車輛，申請逐車少量車型安全審驗且總數未逾二十輛者，得免符合本項「動態煞車」規定。 1.5 同一申請者同一年度同型式規格之M1或N1類車輛申請少量車型安全審驗且總數未逾二十輛或機關、學校進口自行使用之N2、N3類或丙、丁類大客車，得免符合本項「動態煞車」規定中第二煞車系統性能、能量儲存裝置測試及5.2.3.2.3或6.2.3.4.3之電池充能狀態確認。 2. 名詞釋義 … 2.2.27 車輪鎖定（Wheel lock）：指發生打滑率達一．００之狀態。 2.10 緊急煞車作動信號(Emergency Braking Signal)：如5.1.13及6.1.21所述用以表示緊急煞車之邏輯信號。 |
| 2.30. "Yaw rate" means the rate of change of the vehicle's heading angle measured in degrees/ second of rotation about a vertical axis through the vehicle's centre of gravity. 2.32. "Common space" means an area on which more than one tell-tale, indicator, identification symbol, or other message may be displayed but not simultaneously. 2.33. "Static stability factor" means one-half the track width of a vehicle divided by the height of its center of gravity, also expressed as SSF = T/2H, where: T = track width (for vehicles with more than one track width the average is used; for axles with dual wheels, the outer wheels are used when calculating "T") and H = height of the center of gravity of the vehicle. ECE R13 BRAKING Annex21 2010/01/07 11-C2 2.34. "Vehicle Stability Function" means an electronic control function for a vehicle which improves the dynamic stability of the vehicle. 2.34.1. A vehicle stability function includes one or both of the following: (a) directional control (b) roll-over control 2.34.2. Control functions within a vehicle stability function: 2.34.2.1. "Directional control" means a function within a vehicle stability function that assists the driver, in the event of under steer and over steer conditions, within the physical limits of the vehicle in maintaining the direction intended by the driver in the case of a power-driven vehicle, and assists in maintaining the direction of the trailer with that of the towing vehicle in the case of a trailer. 2.34.2.2. "Roll-over control" means a function within a vehicle stability function that reacts to an impending roll-over in order to stabilise the power-driven vehicle or towing vehicle and trailer combination or the trailer during dynamic manoeuvres within the physical limits of the vehicle. |  | 2.17 橫擺角速度（Yaw rate）：係指車頭轉向角在一通過車輛重心之垂直軸旋轉所測得之角度變化率(度/秒)。 2.18 共用空間：係指一可供一個以上之識別標誌、指示器、識別符號或其他資訊顯示之空間，但可不同步顯示。 2.19 靜態穩定因子(SSF)：係指一因子，由車輛一半軌跡之寬度除以車輛重心高度，亦可表示為SSF = T/2H，其中T=軌跡寬度(針對具有一個以上軌跡車種取其平均值，對複輪車軸，計算T時則取外輪之數值)且H=車輛重心高度。 2.20車輛穩定性電子式控制功能（VSF）：係指用電子控制功能提升車輛之動態穩定。 2.20.1車輛穩定性電子式控制功能（VSF）應包含下述其中之一或兩者之功能。 （1）方向性控制 （2）翻覆性控制 2.20.2車輛穩定性電子式控制功能（VSF）之控制功能： 2.20.2.1方向性控制：係指車輛穩定性電子式控制功能（VSF）應能協助駕駛在轉向不足或過度轉向中，於實際條件限制下，動力驅動車輛應能維持其方向控制，及應能協助曳引車聯結拖車之車輛維持行進方向。 2.20.3 翻覆性控制：係指車輛穩定性電子式控制功能（VSF）應能在實際條件限制下，當執行可能產生翻覆之動態操作時，仍能使動力驅動車輛、聯結車輛或拖車穩定不翻覆。 |  |
| Annex 9 ELECTRONIC STABILITY CONTROL SYSTEMS 1. GENERAL REQUIREMENTS Vehicles equipped with an ESC system shall meet the functional requirements specified in paragraph 2. and the performance requirements in paragraph 3. under the test procedures specified in paragraph 4. and under the test conditions specified in paragraph 5. of this annex. 2. FUNCTIONAL REQUIREMENTS Each vehicle to which this annex applies shall be equipped with an electronic stability control system that: 2.1. Is capable of applying braking torques individually to all four wheels 1/ and has a control algorithm that utilizes this capability; 2.2. Is operational over the full speed range of the vehicle, during all phases of driving including acceleration, coasting, and deceleration (including braking), except: 2.2.1. When the driver has disabled ESC; 2.2.2. When the vehicle speed is below 20 km/h; 2.2.3. While the initial start-up self test and plausibility checks are completed, not to exceed 2 minutes when driven under the conditions of paragraph 5.10.2.; 2.2.4. When the vehicle is being driven in reverse. 2.3. Remains capable of activation even if the antilock braking system or traction control system is also activated. |  | 5.5. 車輛穩定性電子式控制系統（ESC） 5.5.1 一般規範 車輛裝設有ESC系統者應符合5.5.2之功能規範，且在5.5.4之測試狀態及5.5.5之測試程序下應符合5.5.3之性能規範。 5.5.2 功能規範 適用5.5規範之車輛應裝設一組車輛穩定性電子式控制系統（ESC），且應符合下述： 5.5.2.1 具備能個別施加煞車扭力至四輪(軸組於測試時視為單軸，複輪則視為單輪)之能力，且擁有能利用該能力之控制演算。 5.5.2.2 在車輛所有速度範圍之車輛駕駛狀態下皆能操作，包含加速、滑行及減速(包含煞車)期間，除： 5.5.2.2.1 當駕駛已關閉ESC。 5.5.2.2.2 當車速低於二０公里/小時。 5.5.2.2.3 在完成初始自我啟動測試及可靠度測試，且於5.5.5.2之狀態下行駛不超過二分鐘。 5.5.2.2.4 當車輛倒車時。 5.5.2.3 即使在防鎖死煞車系統或循跡控制系統啟動時，仍能保有作動能力。 | (5.5.4為測試狀態，5.5.5為測試程序，ECE原文應為誤植) |
| 3. PERFORMANCE REQUIREMENTS During each test performed under the test conditions of paragraph 4. and the test procedure of paragraph 5.9., the vehicle with the ESC system engaged shall satisfy the directional stability criteria of paragraphs 3.1. and 3.2., and it shall satisfy the responsiveness criterion of paragraph 3.3. during each of those tests conducted with a commanded steering wheel 2/ angle of 5A or greater but limited as per paragraph 5.9.4., where A is the steering wheel angle computed in paragraph 5.6.1. Where a vehicle has been physically tested in accordance with paragraph 4. , the compliance of versions or variants of that same vehicle type may be demonstrated by a computer simulation, which respects the test conditions of paragraph 4. and the test procedure of paragraph 5.9. The use of the simulator is defined in Appendix 1 to this annex. 3.1. The yaw rate measured 1 second after completion of the Sine with Dwell steering input (time T0 + 1 in Figure 1) shall not exceed 35 percent of the first peak value of yaw rate recorded after the steering wheel angle changes sign (between first and second peaks) (in Figure 1) during the same test run. |  | 5.5.3 性能規範 在5.5.4之測試狀態及5.5.5.9之測試程序期間，已啟動ESC系統之車輛應滿足5.5.3.1及5.5.3.2之方向穩定標準，且在每一次以受控制之方向盤角度5A或高於但受限於5.5.9.4規範之方向盤角度5A執行測試時，其應滿足5.5.3.3之標準，其中A為5.5.5.6.1所計算出之方向盤角度。 當車輛依照5.5.4完成實際測試，相同車輛型式系列者可使用電腦模擬之方式驗證，其應遵守5.5.4之測試狀態及5.5.5.9之測試程序。模擬方式之採用定義於5.5.6。 5.5.3.1同一測試回合，在方向盤角度變換徵兆之後(在初始及第二極值之間)，當完成正弦轉向輸入(圖六時間T0+1)一秒後，所量測之橫擺角速度（Yaw rate）不得超過初次被記錄橫擺角速度（Yaw rate）極值之百分之三五。 |  |
| Figure 1: Steering wheel position and yaw velocity information used to assess lateral stability. |  | 轉向輪角度 橫擺角速度 時間 時間 圖六 方向盤位置及用於側向穩定度估算法之偏移速度資訊 |  |
| 3.2. The yaw rate measured 1.75 seconds after completion of the Sine with Dwell steering input shall not exceed 20 percent of the first peak value of yaw rate recorded after the steering wheel angle changes sign (between first and second peaks) during the same test run. 3.3. The lateral displacement of the vehicle centre of gravity with respect to its initial straight path shall be at least 1.83 m for vehicles with a GVM of 3,500 kg or less, and 1.52 m for vehicles with a maximum mass greater than 3,500 kg when computed 1.07 seconds after the Beginning of Steer (BOS). BOS is defined in paragraph 5.11.6. 3.3.1. The computation of lateral displacement is performed using double integration with respect to time of the measurement of lateral acceleration at the vehicle centre of gravity, as expressed by the formula: An alternative measuring method may be allowed for type approval testing, provided it demonstrates at least an equivalent level of precision as the double integration method. 3.3.2. Time t = 0 for the integration operation is the instant of steering initiation, known as the Beginning of Steer (BOS). BOS is defined in paragraph 5.11.6. |  | 5.5.3.2同一測試回合，在方向盤角度變換徵兆之後(在初始及第二極值之間)，當完成正弦轉向輸入一‧七五秒後，所量測之橫擺角速度（Yaw rate）不得超過初次被記錄橫擺角速度（Yaw rate）極值之百分之二０。 5.5.3.3在計算5.5.5.11.6所規範之轉向初點(BOS)一‧０七秒後，對於總重量小於或等於三五００公斤之車輛，其車輛重心之橫向位移相對於其初始直線路徑應至少為一‧八三公尺，而對總重量超過三五００公斤者，應至少為一‧五二公尺。 5.5.3.3.1 側向位移之計算是由車輛重心側向加速度與時間雙重積分所求得，公式如下所示： 側向位移=ayC.Gdt 在型式認證測試時允許替代測量方法，惟須確保其準確度至少能與雙重積分法之相當。 5.5.3.3.2 時間t =０之積分為初始轉向之瞬間，此為轉向初點(BOS)，其定義於5.5.5.11.6。 |  |
| 3.4. ESC malfunction detection The vehicle shall be equipped with a tell-tale that provides a warning to the driver of the occurrence of any malfunction that affects the generation or transmission of control or response signals in the vehicle's electronic stability control system. 3.4.1. The ESC malfunction tell-tale: 3.4.1.1. Shall be displayed in direct and clear view of the driver, while in the driver's designated seating position with the driver's seat belt fastened; 3.4.1.2. Shall appear perceptually upright to the driver while driving; 3.4.1.3. Shall be identified by the symbol shown for "ESC Malfunction Tell-tale" below or the text "ESC": 3.4.1.4. Shall be yellow or amber in colour; 3.4.1.5. When illuminated must be sufficiently bright to be visible to the driver under both daylight and night-time driving conditions, when the driver has adapted to the ambient roadway light conditions; 3.4.1.6. Except as provided in paragraph 3.4.1.7., the ESC malfunction tell-tale shall illuminate when a malfunction exists and shall remain continuously illuminated under the conditions specified in paragraph 3.4. for as long as the malfunction exists, whenever the ignition locking system is in the "On" ("Run") position; 3.4.1.7. Except as provided in paragraph 3.4.2., each ESC malfunction tell-tale shall be activated as a check of lamp function either when the ignition locking system is turned to the "On" ("Run") position when the engine is not running, or when the ignition locking system is in a position between "On" ("Run") and "Start" that is designated by the manufacturer as a check position; |  | 5.5.3.4 ESC 故障偵測 車輛應裝設一識別標誌，當車輛之車輛穩定性電子式控制系統（ESC）發生任何會影響控制的產生、傳輸或反應訊號之故障時，提供駕駛人警告。 5.5.3.4.1 ESC 故障識別標誌 5.5.3.4.1.1 當駕駛坐於駕駛座上並將安全帶扣上時，應顯示於駕駛視野直接可見或清晰處。 5.5.3.4.1.2 應能在駕駛開車時直接感知顯示。 5.5.3.4.1.3 應能如下圖鑑別顯示為”ESC 故障識別標誌”或文字”ESC”。 5.5.3.4.1.4 顏色應為黃色或琥珀色。 5.5.3.4.1.5 當發亮時必須能充分明亮至駕駛在白天或夜晚之駕駛狀況皆能看到(當駕駛已適應周遭路面亮度狀況時)。 5.5.3.4.1.6 除5.5.3.4.1.7之情形外，每當點火鎖定系統在”On(Run)”之位置時，若有故障存在則應作動故障識別標誌，且在故障存在期間應持續以5.5.3.4規定之方式發亮。 5.5.3.4.1.7 除5.5.3.4.2之情形外，當引擎未啟動而點火開關系統轉至”On(Run)”，或點火開關系統轉至”On(Run)”及”Start”之間時(申請者設計用以檢查之位置)，每一ESC故障識別標誌皆應作動以檢查功能是否正常。 |  |
| 3.4.1.8. Shall extinguish at the next ignition cycle after the malfunction has been corrected in accordance with paragraph 5.10.4.; 3.4.1.9. May also be used to indicate the malfunction of related systems/functions, including traction control, trailer stability assist, corner brake control, and other similar functions that use throttle and/or individual torque control to operate and share common components with ESC. 3.4.2. The ESC malfunction tell-tale need not be activated when a starter interlock is in operation. 3.4.3. The requirement of paragraph 3.4.1.7. does not apply to tell-tales shown in a common space. 3.4.4. The manufacturer may use the ESC malfunction tell-tale in a flashing mode to indicate ESC operation. |  | 5.5.3.4.1.8 在下一點火循環時，當故障依5.10.4改正後則應熄滅。 5.5.3.4.1.9 亦得用來指示相關系統/功能之異常，包含循跡控制、拖車穩定輔助、角煞車控制及其他類似油門且/或個別扭矩控制操作及與ESC共享元件之功能。 5.5.3.4.2 當啟動器開關於作動狀態時，ESC故障識別標誌不得發亮。 5.5.3.4.3 5.5.3.4.1.7之規範不適用位於共用空間之識別標誌。 5.5.3.4.4 申請者得以閃爍ESC故障識別標誌之模式指示ESC正處於作動中。 |  |
| 3.5. ESC Off and other system controls The manufacturer may include an "ESC Off" control, which shall be illuminated when the vehicle's headlamps are activated, and which has a purpose to place the ESC system in a mode in which it will no longer satisfy the performance requirements of paragraphs 3., 3.1., 3.2. and 3.3. Manufacturers may also provide controls for other systems that have an ancillary effect upon ESC operation. Controls of either kind that place the ESC system in a mode in which it may no longer satisfy the performance requirements of paragraphs 3., 3.1., 3.2. and 3.3. are permitted, provided that the system also meets the requirements of paragraphs 3.5.1., 3.5.2. and 3.5.3. 3.5.1. The vehicle's ESC system shall always return to the manufacturer's original default mode that satisfies the requirements of paragraphs 2. and 3. at the initiation of each new ignition cycle, regardless of what mode the driver had previously selected. However, the vehicle's ESC system need not return to a mode that satisfies the requirements of paragraphs 3. through 3.3. at the initiation of each new ignition cycle if: 3.5.1.1. The vehicle is in a four-wheel drive configuration which has the effect of locking the drive gears at the front and rear axles together and providing an additional gear reduction between the engine speed and vehicle speed of at least 1.6, selected by the driver for lowspeed, off-road driving; or |  | 5.5.3.5 ESC關閉及其他系統控制器 得具有”關閉ESC”之控制器，當車輛頭燈作動且ESC系統不再滿足5.5.3、5.5.3.1、5.5.3.2及5.5.3.3之性能規範時，其應點亮。申請者亦得提供能對其他由ESC作動而輔助之系統之控制功能。任一種類之控制器能使ESC系統不再滿足5.5.3、5.5.3.1、5.5.3.2及5.5.3.3之性能規範時，必須確保系統尚能符合5.5.3.5.1、5.5.3.5.2及5.5.3.5.3之規範。 5.5.3.5.1無論駕駛已事先選擇何種模式，當在每一重新點火之循環時，車輛之ESC系統必須能回復至申請者原始預設模式(須能滿足5.5.2及5.5.3之規範)。然而，當在每一重新點火之循環時，若滿足下述，則車輛之ESC系統可無須回復至能滿足5.5.3至5.5.3.3規範之模式： 5.5.3.5.1.1 若車輛於四輪驅動模式下，其擁有鎖定前後軸之同時驅動的效果，且可於引擎轉速與車輛速度之間提供至少一‧六之額外減速比，則駕駛可選擇低速、越野行駛，或： |  |
| 3.5.1.2. The vehicle is in a four-wheel drive configuration selected by the driver that is designed for operation at higher speeds on snow-, sand-, or dirt-packed roads and that has the effect of locking the drive gears at the front and rear axles together, provided that in this mode the vehicle meets the stability performance requirements of paragraphs 3.1. and 3.2. under the test conditions specified in paragraph 4. However, if the system has more than one ESC mode that satisfies the requirements of paragraphs 3.1. and 3.2. within the drive configuration selected for the previous ignition cycle, the ESC shall return to the manufacturer's original default ESC mode for that drive configuration at the initiation of each new ignition cycle. 3.5.2. A control, whose only purpose is to place the ESC system in a mode in which it will no longer satisfy the performance requirements of paragraphs 3., 3.1., 3.2. and 3.3., shall be identified by the symbol shown for "ESC Off" below or the text "ESC OFF". 3.5.3. A control for an ESC system whose purpose is to place the ESC system in different modes, at least one of which may no longer satisfy the performance requirements of paragraphs 3., 3.1., 3.2., and 3.3., shall be identified by the symbol below with the text "OFF" adjacent to the control position for this mode. Alternatively, in the case where the ESC system mode is controlled by a multifunctional control, the driver display shall identify clearly to the driver the control position for this mode using either the symbol in paragraph 3.5.2. or the text "ESC OFF". 3.5.4. A control for another system that has the ancillary effect of placing the ESC system in a mode in which it no longer satisfies the performance requirements of paragraphs 3., 3.1.,3.2. and 3.3. need not be identified by the "ESC Off" symbol of paragraph 3.5.2. |  | 5.5.3.5.1.2 車輛由駕駛選擇於四輪驅動模式下，設計用於雪地、沙地、或髒污路面之高速操作，其擁有同時鎖定前後軸驅動檔之效果，在5.5.4.之測試狀態下須確保該模式符合5.5.3.1及5.5.3.2之性能規範。然而，若系統可使駕駛在前一點火循環時選擇滿足5.5.3.1及5.5.3.2之一個或以上模式，則ESC必須在每一重新點火循環下回復至申請者原始設定之ESC模式。 5.5.3.5.2若控制器之唯一目的為讓ESC系統模式處於不再滿足5.5.3、5.5.3.1、5.5.3.2及5.5.3.3之性能規範時，應顯示下述”ESC OFF”之符號或文字”ESC OFF”。 5.5.3.5.3若控制器之目的為讓ESC系統處於至少不滿足5.5.3、5.5.3.1、5.5.3.2及5.5.3.3中任一性能規範之不同模式時，應顯示下述符號並於該模式之控制器鄰近處顯示”Off”。 替代作法為，當ESC系統是由複合功能控制器所控制時，應使用5.5.3.5.2之符號或文字”ESC OFF”於儀表板清楚讓駕駛識別其控制模式之位置。 5.5.3.5.4其他系統之控制器若有輔助ESC系統使其模式之效果不再滿足5.5.3、5.5.3.1、5.5.3.2及5.5.3.3之性能規範時，則不需顯示5.5.3.5.2之”ESC關閉”符號。 |  |
| 3.6. ESC OFF tell-tale If the manufacturer elects to install a control to turn off or reduce the performance of the ESC system under paragraph 3.5., the tell-tale requirements of paragraphs 3.6.1. to 3.6.4. shall be met in order to alert the driver to the inhibited or reduced state of ESC system functionality. This requirement does not apply for the driver-selected mode referred to in paragraph 3.5.1.2. 3.6.1. The vehicle manufacturer shall provide a tell-tale indicating that the vehicle has been put into a mode that renders it unable to satisfy the requirements of paragraphs 3., 3.1., 3.2. and 3.3., if such a mode is provided. 3.6.2. The "ESC Off" tell-tale: 3.6.2.1. Shall be displayed in direct and clear view of the driver while in the driver's designated seating position with the driver's seat belt fastened; 3.6.2.2. Shall appear perceptually upright to the driver while driving; 3.6.2.3. Shall be identified by the symbol shown for "ESC Off" below or the text "ESC OFF"; or Shall be identified with the English word "OFF" adjacent to either the control referred to in paragraph 3.5.2. or 3.5.3. or the illuminated malfunction tell-tale; 3.6.2.4. Shall be yellow or amber in colour; 3.6.2.5. When illuminated, shall be sufficiently bright to be visible to the driver under both daylight and night time driving conditions, when the driver has adapted to the ambient roadway light conditions; 3.6.2.6. Shall remain continuously illuminated for as long as the ESC is in a mode that renders it unable to satisfy the requirements of paragraphs 3., 3.1., 3.2. and 3.3; 3.6.2.7. Except as provided in paragraphs 3.6.3. and 3.6.4. each "ESC Off" tell-tale shall be activated as a check of lamp function either when the ignition locking system is turned to the "On" ("Run") position when the engine is not running, or when the ignition locking system is in a position between "On" ("Run") and "Start" that is designated by the manufacturer as a check position. |  | 5.5.3.6 ESC關閉識別標誌 若申請者選擇裝設一控制器以關閉或減少5.5.3.5規範下ESC系統之性能，則其識別標誌應能符合5.5.3.6.1至5.5.3.6.4之規範以警告駕駛者ESC系統之機能受限制或降低。此規範不適用5.5.3.5.1.2所述之駕駛者選擇模式。 5.5.3.6.1若具備一可使車輛無法滿足5.5.3、5.5.3.1、5.5.3.2及5.5.3.3之模式，則申請者應提供一指示此模式之識別標誌。 5.5.3.6.2 「ESC OFF」之識別標誌： 5.5.3.6.2.1當駕駛坐於駕駛座上並將安全帶扣上時，應顯示於駕駛視野直接可見或清晰處。 5.5.3.6.2.2 應能在駕駛開車時直接感知顯示。 5.5.3.6.2.3 應能如下圖鑑別顯示為”ESC 關閉”或文字”ESC OFF”。 或應能於5.5.3.5.2或5.5.3.5.3所述功能之控制器鄰近處顯示”OFF” 或點亮故障識別標誌。 5.5.3.6.2.4 顏色應為黃色或琥珀色。 5.5.3.6.2.5 當發亮時必須能充分明亮至駕駛在白天或夜晚之駕駛狀況皆能看到(當駕駛已適應周遭路面亮度狀況時)。 5.5.3.6.2.6當車輛處於無法滿足5.5.3、5.5.3.1、5.5.3.2及5.5.3.3之模式時，識別標誌應持續點亮。 5.5.3.6.2.7 除5.5.3.6.3及5.5.3.6.4之情形外，當引擎未啟動而點火開關系統轉至”On(Run)”，或當點火開關系統轉至”On(Run)”及”Start”之間時(申請者設計用以檢查之位置)，每一ESC關閉識別標誌皆應作動以檢查功能是否正常。 |  |
| 3.6.2.8. Shall extinguish after the ESC system has been returned to the manufacturer's original default mode. 3.6.3. The "ESC Off" tell-tale need not be activated when a starter interlock is in operation. 3.6.4. The requirement of paragraph 3.6.2.7. of this annex does not apply to tell-tales shown in a common space. 3.6.5. The manufacturer may use the "ESC Off" telltale to indicate an ESC level of function other than the manufacturer's original default mode even if the vehicle would meet paragraphs 3., 3.1., 3.2. and 3.3. of this annex at that level of ESC function. |  | 5.5.3.2.6.8 在ESC系統回復至申請者原始設定模式後燈號應熄滅。 5.5.3.6.3當啟動器開關於作動狀態時，ESC關閉識別標誌不得發亮。 5.5.3.6.4 5.5.3.6.2.7之規範不適用位於共用空間之識別標誌。 5.5.3.6.5 申請者可使用”ESC OFF”識別標誌以指示有別於申請者原始設定模式之ESC功能層級，即使該車輛尚能符合5.5.3、5.5.3.1、5.5.3.2及5.5.3.3之ESC功能層級規範。 |  |
| 4. TEST CONDITIONS 4.1. Ambient conditions 4.1.1. The ambient temperature is between 0 degrees C and 45 degrees C. 4.1.2. The maximum wind speed is no greater than 10 m/s for vehicles with SSF > 1.25, and 5 m/s for vehicles with SSF < 1.25. 4.2. Road test surface 4.2.1. Tests are conducted on a dry, uniform, solid-paved surface. Surfaces with irregularities and undulations, such as dips and large cracks, are unsuitable. 4.2.2. The road test surface has a nominal 3/ peak braking coefficient (PBC) of 0.9, unless otherwise specified, when measured using either: 4.2.2.1. The American Society for Testing and Materials (ASTM) E1136 standard reference test tyre, in accordance with ASTM Method E1337-90, at a speed of 40 mph; or 4.2.2.2. The k-test method specified in Appendix 2 to Annex 6 of this Regulation. 3/ The "nominal" value is understood as being the theoretical target value. 4.2.3. The test surface has a consistent slope between level and 1 per cent. 4.3. Vehicle conditions 4.3.1. The ESC system is enabled for all testing. 4.3.2. Vehicle mass. The vehicle is loaded with the fuel tank filled to at least 90 per cent of capacity, and a total interior load of 168 kg comprised of the test driver, approximately 59 kg of test equipment (automated steering machine, data acquisition system and the power supply for the steering machine), and ballast as required to make up for any shortfall in the weight of test drivers and test equipment. Where required, ballast shall be placed on the floor behind the passenger front seat or if necessary in the front passenger foot well area. All ballast shall be secured in a way that prevents it from becoming dislodged during testing. 4.3.3. Tyres. The tyres are inflated to the vehicle manufacturer's recommended cold inflation pressure(s) e.g. as specified on the vehicle's placard or the tyre inflation pressure label. Tubes may be installed to prevent tyre de-beading. |  | 5.5.4. 測試狀態 5.5.4.1 環境狀態 5.5.4.1.1 環境溫度在攝氏０度至四五度間。 5.5.4.1.2 當車輛靜態穩定因子(SSF) 大於一‧二五時，其最大風速不得高於一０公尺/秒，而當車輛靜態穩定因子(SSF) 小於一‧二五時，其最大風速不得高於五公尺/秒。 5.5.4.2 測試路面之鋪面 5.5.4.2.1 應在乾燥、平整及堅硬之路面執行測試，路面不得有不平整及高低起伏之狀況(如坑洞或大裂縫)。 5.5.4.2.2除非另有規定，否則測試路面鋪面之額定最高煞車係數（Peak braking coefficient (PBC)）應達０‧九，並依照下述之一進行量測： 5.5.4.2.2.1使用美國測試及材料協會(ASTM) E1136標準之參考試驗胎，依據ASTM E1337-90，在速度每小時四０英哩乾燥之路面，或 5.5.4.2.2.2依本基準項次「四十三之一」6.2.5.1所規範之方法。 5.5.4.2.3 測試表面其坡度應介於０至百分之一間。 5.5.4.3 車輛狀態 5.5.4.3.1 ESC系統應能在所有測試中作動。 5.5.4.3.2 車輛重量 車輛應至少負載至其油箱容量百分之九０且總內部負載為一六八公斤，其由測試駕駛員、接近五九公斤之測試設備(自動轉向機、數據截取系統及轉向機之電源供應器)所構成，亦包含任何彌補駕駛員或測試設備不足重量之配重。當需要時，配重應置放於副駕駛座座椅後方，或必要時放於前方乘客腳踏處。所有配重皆應確保不會在測試期間滑動。 5.5.4.3.3 輪胎 輪胎必須充氣至申請者建議之胎壓，如車輛之標籤或胎壓標籤所標示。可裝入內胎以避免輪胎失壓。 |  |
| 4.3.4. Outriggers. Outriggers may be used for testing if deemed necessary for test drivers' safety. In this case, the following applies for vehicles with a Static Stability Factor (SSF)< 1.25: 4.3.4.1. Vehicles with a mass in running order under 1,588 kg shall be equipped with "lightweight" outriggers. Lightweight outriggers shall be designed with a maximum mass of 27 kg and a maximum roll moment of inertia of 4.3.4.2. Vehicles with a mass in running order between 1,588 kg and 2,722 kg shall be equipped with "standard" outriggers. Standard outriggers shall be designed with a maximum mass of 32 kg and a maximum roll moment of inertia of 4.3.4.3. Vehicles with a mass in running order equal to or greater than 2,722 kg shall be equipped with "heavy" outriggers. Heavy outriggers shall be designed with a maximum mass of 39 kg and a maximum roll moment of inertia of 4.3.5. Automated steering machine. A steering robot programmed to execute the required steering pattern shall be used in paragraphs 5.5.2., 5.5.3., 5.6. and 5.9. The steering machine shall be capable of supplying steering torques between 40 to 60 Nm. The steering machine shall be able to apply these torques when operating with steering wheel velocities up to 1,200 degrees per second. |  | 5.5.4.3.4支撐架 為確保測試員之安全，得使用支撐架。下述規範適用於車輛靜態穩定因子(SSF)小於一‧二五時。 5.5.4.3.4.1 對於可行駛狀態之車重低於一五八八公斤者，應裝設”輕量化”支撐架。輕量化支撐架最大設計重量為二七公斤且其最大轉動慣性矩為27kg⋅m2。 5.5.4.3.4.2 對於可行駛狀態之車重介於一五八八公斤至二七二二公斤者，應裝設”標準”支撐架。標準支撐架最大設計重量為三二公斤且其最大轉動慣性矩為35.9kg⋅m2。 5.5.4.3.4.3對於可行駛狀態之車重大於二七二二公斤者，應裝設”重型”支撐架。重型支撐架最大設計重量為三九公斤且其最大轉動慣性矩為40.7kg⋅m2。 5.5.4.3.5 自動轉向機 應使用可依5.5.5.5.2、5.5.5.5.3、5.5.5.6及5.5.5.9之要求執行轉向模式測試之機械轉向臂。轉向機應能提供介於四０至六０Nm間之轉向力矩。即使當方向盤運作速度達每秒一二００度時，此轉向機應仍能施加上述力矩。 |  |
| 5. Test Procedure 5.1. Inflate the vehicles' tyres to the manufacturer's recommended cold inflation pressure(s) e.g. as provided on the vehicle's placard or the tyre inflation pressure label. 5.2. Tell-tale bulb check. With the vehicle stationary and the ignition locking system in the "Lock" or "Off" position, switch the ignition to the "On" ("Run") position or, where applicable, the appropriate position for the lamp check. The ESC malfunction tell-tale shall be illuminated as a check of lamp function, as specified in paragraph 3.4.1.7., and if equipped, the "ESC Off" tell-tale shall also be illuminated as a check of lamp function, as specified in paragraph 3.6.2.7. The tell-tale bulb check is not required for a tell-tale shown in a common space as specified in paragraphs 3.4.3. and 3.6.4. 5.3. "ESC Off" control check. For vehicles equipped with an "ESC Off" control, with the vehicle stationary and the ignition locking system in the "Lock" or "Off" position, switch the ignition locking system to the "On" ("Run") position. Activate the "ESC Off" control and verify that the "ESC Off" tell-tale is illuminated, as specified in paragraph 3.6.2. Turn the ignition locking system to the "Lock" or "Off" position. Again, switch the ignition locking system to the "On" ("Run") position and verify that the "ESC Off" telltale has extinguished indicating that the ESC system has been restored as specified in paragraph 3.5.1. |  | 5.5.5. 測試程序 5.5.5.1將輪胎充氣至申請者建議之胎壓，如車輛之標籤或胎壓標籤所標示。 5.5.5.2 識別標誌燈泡檢查 當車輛靜止且點火開關在”Lock”或”Off”之位置時，將其切換至”On(Run)”，或其他適當之位置，以檢查識別標誌之燈號。ESC故障識別標誌在燈號功能檢查時應如5.5.3.4.1.7所規定發亮，且若有裝設ESC關閉識別標誌則在燈號功能檢查時亦應如5.5.3.6.2.7所規定發亮。本項識別標誌燈泡檢查不適用於5.5.3.4.3及5.5.3.6.4所規範在共用空間之識別標誌。 5.5.5.3 “ESC OFF”控制器確認 對裝設有ESC關閉控制器之車輛，當車輛靜止且點火開關在”Lock”或”Off”之位置時，將其切換至”On(Run)”之位置。作動”ESC OFF”控制器並確認ESC關閉識別標誌是否如5.5.3.6.2所規定發亮。 將點火開關回復至”Lock”或”Off”之位置，並再次切換至”On(Run)”之位置，確認ESC關閉識別標誌已熄滅，並如5.5.3.5.1所規範將ESC系統重置。 |  |
| 5.4. Brake conditioning Condition the vehicle brakes in the manner described in paragraphs 5.4.1. to 5.4.4. 5.4.1. Ten stops are performed from a speed of 56 km/h, with an average deceleration of approximately 0.5g. 5.4.2. Immediately following the series of ten 56 km/h stops, three additional stops are performed from 72 km/h at higher deceleration. 5.4.3. When executing the stops in paragraph 5.4.2., sufficient force is applied to the brake pedal to bring the vehicle's antilock braking system (ABS) into operation for a majority of each braking event. 5.4.4. Following completion of the final stop in 5.4.2., the vehicle is driven at a speed of 72 km/h for five minutes to cool the brakes. |  | 5.5.5.4 煞車調節程序 依照5.5.5.4.1至5.5.5.4.4之程序作動車輛煞車。 5.5.5.4.1自車速五六公里/小時以平均減速度為０‧五g之方式執行一０次煞停。 5.5.5.4.2 接著依照五六公里/小時煞車之方法，執行三次較高減速度之車速七二公里/小時煞停。 5.5.5.4.3 當執行5.5.5.4.2之測試時，應給予煞車踏板足夠之施力，以於多數之煞停程序中能使防鎖死煞車系統作動。 5.5.5.4.4 完成5.5.5.4.2之最後一次煞停後，車輛須以七二公里/小時之速度行駛五分鐘以冷卻煞車。 |  |
| 5.5. Tyre Conditioning Condition the tyres using the procedure of paragraphs 5.5.1. to 5.5.3. to wear away mould sheen and achieve operating temperature immediately before beginning the test runs of paragraphs 5.6. and 5.9. 5.5.1. The test vehicle is driven around a circle 30 meters in diameter at a speed that produces a lateral acceleration of approximately 0.5 to 0.6g for three clockwise laps followed by three anticlockwise laps. 5.5.2. Using a sinusoidal steering pattern at a frequency of 1 Hz, a peak steering wheel angle amplitude corresponding to a peak lateral acceleration of 0.5 to 0.6g, and a vehicle speed of 56 km/h, the vehicle is driven through four passes performing 10 cycles of sinusoidal steering during each pass. 5.5.3. The steering wheel angle amplitude of the final cycle of the final pass shall be twice that of the other cycles. The maximum time permitted between each of the laps and passes is five minutes. |  | 5.5.5.5 輪胎調節程序 依照5.5.5.5.1至5.5.5.5.3之程序磨除輪胎之毛邊且於開始5.5.5.6至5.5.5.9之測試前達到工作溫度。 5.5.5.5.1 測試車輛沿直徑三０公尺之圓環並以能產生接近０‧五至０‧六g側向加速度之速度繞行，先以順時針方向行駛三圈接著以逆時針方向行駛三圈。 5.5.5.5.2 使用頻率一Hz之正弦轉向模式，與符合最高側向加速度０‧五至０‧六g之方向盤轉角振幅極值，且車速為五六公里/小時，使車輛繞行四次，並於每次進行一０次之正弦轉向循環。 5.5.5.5.3 最終次之最終循環之方向盤轉角振幅應為其他循環之二倍。在每圈及每次之最大允許時間間格為五分鐘。 |  |
| 5.6. Slowly increasing steer procedure The vehicle is subjected to two series of runs of the slowly increasing steer test using a constant vehicle speed of 80 +/- 2 km/h and a steering pattern that increases by 13.5 degrees per second until a lateral acceleration of approximately 0.5g is obtained. Three repetitions are performed for each test series. One series uses anticlockwise steering, and the other series uses clockwise steering. The maximum time permitted between each test run is five minutes. 5.6.1. From the slowly increasing steer tests, the quantity "A" is determined. "A" is the steering wheel angle in degrees that produces a steady state lateral acceleration (corrected using the methods specified in paragraph 5.11.3.) of 0.3g for the test vehicle. Utilizing linear regression, A is calculated, to the nearest 0.1 degrees, from each of the six slowly increasing steer tests. The absolute value of the six A values calculated is averaged and rounded to the nearest 0.1 degrees to produce the final quantity, A, used below. 5.7. After the quantity A has been determined, without replacing the tyres, the tyre conditioning procedure described in paragraph 5.5. is performed again immediately prior to conducting the Sine with Dwell test of paragraph 5.9. Initiation of the first Sine with Dwell test series shall begin within two hours after completion of the slowly increasing steer tests of paragraph 5.6. |  | 5.5.5.6緩慢增加轉向程序 車輛應符合兩系列之緩慢增加轉向程序系列規範，其使用車輛常速為八０正負二公里/小時且每秒鐘以一三‧五度逐漸增加之轉向模式直到獲得０‧五g之側向加速度。每一系列之測試應執行三次。一系列使用逆時針轉向，另一系列使用順時針轉向。在每次測試之最大允許間隔時間為五分鐘。 5.5.5.6.1 應從緩慢增加轉向測試中量測”A”之值。”A”為能讓車輛產生穩態側向加速度(以5.5.5.11.3之方法校正)０‧三g之方向盤角度(單位度)。利用線性回歸，可從此六次緩慢增加轉向測試計算出A(至最小角度０‧一度)。將此六次A值取平均值並進位至最小為０‧一度時即為其絕對值”A”。 5.5.5.7 在A被決定後，不更換輪胎，並於進行5.5.5.9之正弦轉向測試前依5.5.5.5規範再次進行輪胎調節程序。應在完成5.5.5.6規範之緩慢增加轉向程序後二小時內開始首次之正弦轉向測試。 |  |
| 5.8. Check that the ESC system is enabled by ensuring that the ESC malfunction and "ESC Off" (if provided) tell-tales are not illuminated. 5.9. Sine with Dwell test of oversteer intervention and responsiveness The vehicle is subjected to two series of test runs using a steering pattern of a sine wave at 0.7 Hz frequency with a 500 ms delay beginning at the second peak amplitude as shown in Figure 2 (the Sine with Dwell tests). One series uses anticlockwise steering for the first half cycle, and the other series uses clockwise steering for the first half cycle. The vehicle is allowed to cool-down between each test runs for a period of 1.5 to 5 minutes, with the vehicle stationary. |  | 5.5.5.8 藉由ESC故障及ESC關閉(若有提供)識別標誌未點亮確認ESC系統能運作。 5.5.5.9 轉向過度之干預及回應之正弦轉向測試 車輛應執行兩系列之測試，使用頻率0.7Hz與500ms延遲時間並自第二次極值後開始之轉向模式(如圖七所示)。第一系列在初次半圈使用逆時針轉向，其他系列在初次半圈使用順時針轉向。車輛允許在每次測試期間靜止冷卻一‧五至五分鐘間。 |  |
| Figure 2: Sine with Dwell steering profile |  | 轉向輪角度 時間 圖七、正弦轉向概要 |  |
| 5.9.1. The steering motion is initiated with the vehicle coasting in high gear at 80 +/- 2 km/h. 5.9.2. The steering amplitude for the initial run of each series is 1.5 A, where A is the steering wheel angle determined in paragraph 5.6.1. 5.9.3. In each series of test runs, the steering amplitude is increased from run to run, by 0.5 A, provided that no such run will result in a steering amplitude greater than that of the final run specified in paragraph 5.9.4. 5.9.4. The steering amplitude of the final run in each series is the greater of 6.5 A or 270 degrees, provided the calculated magnitude of 6.5 A is less than or equal to 300 degrees. If any 0.5 A increment, up to 6.5 A, is greater than 300 degrees, the steering amplitude of the final run shall be 300 degrees. 5.9.5. Upon completion of the two series of test runs, post processing of yaw rate and lateral acceleration data is done as specified in paragraph 5.11. |  | 5.5.5.9.1 當車輛於八０正負二公里／小時之高速檔時，開始作動轉向。 5.5.5.9.2 每一系列測試之初始轉向幅度為一‧五A，其中A為5.5.5.6.1所決定之轉向幅度角。 5.5.5.9.3 在每一測試系列時，轉向幅度應以０‧五A隨著測試逐次增加，並確保不應有超過5.5.5.9.4之轉向幅度。 5.5.5.9.4 每系列之最終測試轉向幅度為大於六‧五A或二七０度，並應確保六‧五A之計算強度小於或等於三００度。若有任何以０‧五A定量增加至六‧五A並超過三００度者，其最終轉向幅度應為三００度。 5.5.5.9.5 在完成此二系列之測試後，應依照5.5.5.11之規範算出橫擺角速度（Yaw rate）及側向加速度。 |  |
| 5.10. ESC malfunction detection 5.10.1. Simulate one or more ESC malfunction(s) by disconnecting the power source to any ESC component, or disconnecting any electrical connection between ESC components (with the vehicle power off). When simulating an ESC malfunction, the electrical connections for the tell-tale lamp(s) and/or optional ESC system control(s) are not to be disconnected. 5.10.2. With the vehicle initially stationary and the ignition locking system in the "Lock" or "Off" position, switch the ignition locking system to the "Start" position and start the engine. Drive the vehicle forward to obtain a vehicle speed of 48 +/- 8 km/h. 30 seconds, at the latest, after the engine has been started and within the next two minutes at this speed, conduct at least one left and one right smooth turning manoeuvre without losing directional stability and one brake application. Verify that the ESC malfunction indicator illuminates in accordance with paragraph 3.4. by the end of these manoeuvres. 5.10.3. Stop the vehicle, switch the ignition locking system to the "Off" or "Lock" position. After a five-minute period, switch the vehicle's ignition locking system to the "Start" position and start the engine. Verify that the ESC malfunction indicator again illuminates to signal a malfunction and remains illuminated as long as the engine is running or until the fault is corrected. 5.10.4. Switch the ignition locking system to the "Off" or "Lock" position. Restore the ESC system to normal operation, switch the ignition system to the "Start" position and start the engine. Re-perform the manoeuvre described in paragraph 5.10.2. and verify that the tell-tale has extinguished within this time or immediately afterwards. |  | 5.5.5.10 ESC故障偵測 5.5.5.10.1 藉由切斷任何ESC元件之電力供應，或切斷任何ESC元件間之電子連結(車輛為熄火狀態)，以模擬一個或多個ESC故障。當模擬ESC故障時，識別標誌之電子連結及/或選用ESC系統之控制器之連結不應被截斷。 5.5.5.2 當車輛為靜止且點火開關裝置於"Lock" 或"Off"之位置時，切換點火開關裝置至"Start"之位置且發動引擎。往前駕駛車輛以獲得四八正負八公里/小時之車速。在引擎發動後三０秒內且在達到該車速後二分鐘內，執行至少一次向左及一次向右之平滑轉向操作(不得喪失方向穩定度)及作動一次煞車。在這一系列操作後確認ESC故障指示燈依照5.5.3.4之規範發亮。 5.5.5.10.3停止車輛，並切換點火開關系統至”Off" 或 "Lock"之位置。在五分鐘後，切換車輛點火開關系統至"Start"之位置並發動引擎。再次確認ESC故障指示燈發亮以警示故障且能於引擎運轉期間持續發亮或至失效被修正為止。 5.5.5.10.4 切換點火開關系統至”Off" 或 "Lock"之位置。重置ESC系統至正常狀態，切換點火開關系統至"Start"之位置且發動引擎。重新執行5.5.5.10.2之操作且確認識別標誌在該次時間內熄滅或已立刻熄滅。 |  |
| 5.11. Post data processing - calculations for performance metrics Yaw rate and lateral displacement measurements and calculations shall be processed utilizing the techniques specified in paragraphs 5.11.1. to 5.11.8. 5.11.1. Raw steering wheel angle data is filtered with a 12-pole phaseless Butterworth filter and a cut-off frequency of 10 Hz. The filtered data is then zeroed to remove sensor offset utilizing static pre-test data. 5.11.2. Raw yaw rate data is filtered with a 12-pole phaseless Butterworth filter and a cut-off frequency of 6 Hz. The filtered data is then zeroed to remove sensor offset utilizing static pre-test data. 5.11.3. Raw lateral acceleration data is filtered with a 12-pole phaseless Butterworth filter and a cut-off frequency of 6 Hz. The filtered data is then zeroed to remove sensor offset utilizing static pre-test data. The lateral acceleration data at the vehicle centre of gravity is determined by removing the effects caused by vehicle body roll and by correcting for sensor placement via the use of coordinate transformation. For data collection, the lateral accelerometer shall be located as close as possible to the position of the vehicle's longitudinal and lateral centres of gravity. |  | 5.5.5.11 數據後處理-計算性能規律 橫擺角速度（Yaw rate）及側向位移之量測與計算應依照5.5.5.11.1至5.5.5.11.8之方法處理。 5.5.5.11.1 原始方向盤轉角資料係由12-pole phaseless巴特沃思濾波器及一０Hz之界限頻率而過濾。然後利用靜態預測試之資料將所過濾之資料進行歸零補償以移除感應器偏移。 5.5.5.11.2 原始橫擺角速度（Yaw rate）資料係由12-pole phaseless巴特沃思濾波器及六Hz之界限頻率而過濾。然後利用靜態預測試之資料將所過濾之資料進行歸零補償以移除感應器偏移。 5.5.5.11.3 原始側向加速度資料係由12-pole phaseless巴特沃思濾波器及六Hz之界限頻率而過濾。然後利用靜態預測試之資料將所過濾之資料進行歸零補償以移除感應器偏移。車輛重心處之側向加速度資料係藉由移除車輛打滑所造成之效果及利用座標轉換來校正感應器之位移而得。 為利收集數據，側向加速計應儘可能置放於車輛縱向及側向之重心處。 |  |
| 5.11.4. Steering wheel velocity is determined by differentiating the filtered steering wheel angle data. The steering wheel velocity data is then filtered with a moving 0.1 second running average filter. 5.11.5. Lateral acceleration, yaw rate and steering wheel angle data channels are zeroed utilizing a defined "zeroing range." The methods used to establish the zeroing range are defined in paragraphs 5.11.5.1. and 5.11.5.2. 5.11.5.1. Using the steering wheel rate data calculated using the methods described in paragraph 5.11.4., the first instant that the steering wheel rate exceeds 75 deg/sec is identified. From this point, steering wheel rate shall remain greater than 75 deg/sec for at least 200 ms. If the second condition is not met, the next instant that the steering wheel rate exceeds 75 deg/sec is identified and the 200 ms validity check applied. This iterative process continues until both conditions are ultimately satisfied. 5.11.5.2. The "zeroing range" is defined as the 1.0 second time period prior to the instant the steering wheel rate exceeds 75 deg/sec (i.e., the instant the steering wheel velocity exceeds 75 deg/sec defines the end of the "zeroing range"). 5.11.6. The Beginning of Steer (BOS) is defined as the first instance when the filtered and zeroed steering wheel angle data reaches -5 degrees (when the initial steering input is anticlockwise) or +5 degrees (when the initial steering input is clockwise) after a time defining the end of the "zeroing range." The value for time at the BOS is interpolated. |  | 5.5.5.11.4 方向盤速度係由微分所過濾之方向盤角度資料而得。方向盤速度資料由每０‧一秒移動之均值濾波器取得。 5.5.5.11.5 側向加速度、橫擺角速度（Yaw rate）及方向盤角度資料波段係利用”歸零範圍”歸零。 此方法定義於5.5.5.11.5.1及5.5.5.11.5.2 5.5.5.11.5.1 使用5.5.5.11.4之方法計算方向盤轉向率，一旦方向盤轉向率超過七五度/秒則應被識別。從此點開始，方向盤轉向率應維持超過七五度/秒至少二００微秒。若不符合第二狀況，則當下一刻識別出方向盤轉向率超過七五度/秒時應確認是否維持至少二００微秒。重覆此過程並持續至兩者情況全部滿足為止。 5.5.5.11.5.2 “歸零範圍”之定義為當識別出方向盤轉向率超過七五度/秒瞬間之前一秒(亦即超過七五度/秒之方向盤轉向率之瞬間，此即為歸零之結束點)。 5.5.5.11.6 轉向初點(BOS)係定義為當歸零結束時，方向盤轉角經過濾及歸零後於第一瞬間達到負五度(當初始轉向輸入為逆時針)或正五度(當初始轉向輸入為順時針)時。BOS之時間值以內插法取得。 |  |
| 5.11.7. The Completion of Steer (COS) is defined as the time the steering wheel angle returns to zero at the completion of the Sine with Dwell steering manoeuvre. The value for time at the zero degree steering wheel angle is interpolated. 5.11.8. The second peak yaw rate is defined as the first local yaw rate peak produced by the reversal of the steering wheel. The yaw rates at 1.000 and 1.750 seconds after COS are determined by interpolation. 5.11.9. Determine lateral velocity by integrating corrected, filtered and zeroed lateral acceleration data. Zero lateral velocity at the BOS point. Determine lateral displacement by integrating zeroed lateral velocity. Zero lateral displacement at the BOS point. The lateral displacement measurement is made at 1.07 seconds after BOS point and is determined by interpolation. |  | 5.5.5.11.7 轉向完成(COS)係定義為完成正弦轉向操作後當方向盤轉向角回復至零度之時間。方向盤轉角回復至零度之時間值以內差法取得。 5.5.5.11.8 第二橫擺角速度（Yaw rate）係定義為當反轉方向盤時所產生之初次局部橫擺角速度（Yaw rate）極值。COS後一秒及一‧七五秒之橫擺角速度（Yaw rate）係以內插法取得。 5.5.5.11.9 藉由積分校正過濾及歸零後之側向加速度數據以決定側向速度。在BOS點時歸零側向速度。藉由積分歸零側向速度以決定側向位移。在BOS點時歸零側向位移。側向位移在BOS點後一‧０七秒藉內插法決定。 |  |
| Annex 9 - Appendix 1 USE OF THE DYNAMIC STABILITY SIMULATION The effectiveness of the electronic stability control system may be determined by computer simulation. 1. USE OF THE SIMULATION 1.1. The vehicle stability function shall be demonstrated by the vehicle manufacturer to the Type Approval Authority or Technical Service by simulating the dynamic manoeuvres of paragraph 5.9. of Annex 9. 1.2. The simulation shall be a means whereby the vehicle stability performance shall be demonstrated with: (a) The yaw rate, one second after completion of the Sine with Dwell steering input (time T0 + 1); (b) The yaw rate, 1.75 seconds after completion of the Sine with Dwell steering input; (c) The lateral displacement of the vehicle centre of gravity with respect to its initial straight path. 1.3. The simulation shall be carried out with a validated modelling and simulation tool and using the dynamic manoeuvres of paragraph 5.9. of Annex 9 under the test conditions of paragraph 4. of Annex 9. The method by which the simulation tool is validated is given in Appendix 2 to this annex. |  | 5.5.6 動態穩定模擬：車輛穩定性電子式控制系統（ESC）之效能可藉由電腦模擬之方式決定。 5.5.6.1 模擬方法之使用 5.5.6.1.1 車輛之穩定功能應由申請者以模擬5.5.5.9之動態操作展示予檢測機構。 5.5.6.1.2 此模擬應藉由可展現下述車輛穩定性能之方式進行： (a)在完成正弦轉向輸入後一秒(T0+1)之橫擺角速度（Yaw rate）。 (b)在完成正弦轉向輸入後一‧七五秒之橫擺角速度（Yaw rate）。 (c)相對於車輛初始直線路徑之車輛重心側向位移。 5.5.6.1.3 此模擬應使用有效之模型及模擬工具且使用5.5.4之測試狀態與5.5.5.9之動態操作方法。有效之模擬方法如5.5.7所述。 |  |
| Annex 9 - Appendix 2 DYNAMIC STABILITY SIMULATION TOOL AND ITS VALIDATION 1. SPECIFICATION OF THE SIMULATION TOOL 1.1. The simulation method shall take into account the main factors which influence the directional and roll motion of the vehicle. A typical model may include the following vehicle parameters in an explicit or implicit form: (a) Axle/wheel; (b) Suspension; (c) Tyre; (d) Chassis/vehicle body; (e) Power train/driveline, if applicable; (f) Brake system; (g) Pay load. 1.2. The Vehicle Stability Function shall be added to the simulation model by means of: (a) A subsystem (software model) of the simulation tool; or (b) The electronic control box in a hardware-in-the-loop configuration. 2. VALIDATION OF THE SIMULATION TOOL 2.1. The validity of the applied modelling and simulation tool shall be verified by means of comparisons with practical vehicle tests. The tests utilised for the validation shall be the dynamic manoeuvres of paragraph 5.9. of Annex 9. During the tests, the following motion variables, as appropriate, shall be recorded or calculated in accordance with ISO 15037 Part 1:2005: General conditions for passenger cars or Part 2:2002: General conditions for heavy vehicles and buses (depending on the vehicle category): (a) Steering-wheel angle (delta H); (b) Longitudinal velocity (vX); (c) Sideslip angle (beta) or lateral velocity (vY);(optional); (d) Longitudinal acceleration (aX); (optional); (e) Lateral acceleration (aY); (f) Yaw velocity (d psi/dt); (g) Roll velocity (d phi/dt); (h) Pitch velocity (d theta/dt); (i) Roll angle (phi); (j) Pitch angle (theta). 2.2. The objective is to show that the simulated vehicle behaviour and operation of the vehicle stability function is comparable with that seen in practical vehicle tests. 2.3. The simulator shall be deemed to be validated when its output is comparable to the practical test results produced by a given vehicle type during the dynamic manoeuvres of paragraph 5.9. of Annex 9. The relationship of activation and sequence of the vehicle stability function in the simulation and in the practical vehicle test shall be the means of making the comparison. 2.4. The physical parameters that are different between the reference vehicle and simulated vehicle configurations shall be modified accordingly in the simulation. 2.5. A simulator test report shall be produced, a model of which is defined in Appendix 3 to this annex, and a copy attached to the vehicle approval report. |  | 5.5.7 動態穩定模擬工具及其有效性 5.5.7.1 模擬工具之規格 5.5.7.1.1 模擬方法應考量會影響車輛行進方向與打滑之主要因子。典型的模型可包含下述車輛參數： (a)車軸/車輪； (b)懸吊； (c)輪胎； (d)底盤/車身； (e)動力系統/驅動線(若有的話)； (f)煞車系統； (g)車身負載。 5.5.7.1.2車輛穩定性電子式控制功能（VSF）應以下述方式加入模擬模型中： (a)模擬工具之子系統(軟體模型)；或 (b)於硬體控制迴圈配置中增加電子控制單元。 5.5.7.2 模擬工具之有效性 5.5.7.2.1 應藉由比較車輛實際測試結果來驗證所使用模型及模擬工具之有效性。以5.5.5.9之動態操作作為驗證有效性之測試方法。 於測試期間，下述運動參數應視實際適當狀況予以記錄或依照ISO 15037 Part1:2005(小客車之一般狀態)或Part2 :2002(大型車輛之一般狀態)計算(視車輛種類而定)。 (a)方向盤角度(delta H); (b)縱向速度(vX); (c)側滑角(beta)或側向速度(vY);(選用); (d)縱向加速度(aX); (選用); (e)側向加速度(aY); (f)偏移角速度(d psi/dt); (g)轉動角速度(d phi/dt); (h)俯仰角速度(d theta/dt); (i)轉動角 (phi); (j)俯仰角 (theta). 5.5.7.2.2 以呈現被模擬車輛之行為及車輛穩定功能之操作與實際車輛測試結果可相當為目標。 5.5.7.2.3 對於所模擬車型依5.5.5.9動態操作方法所得到之輸出與實際車輛測試結果可相當，則此模擬器可視為有效。在模擬時應使用可與實際車輛測試相當之方法模擬車輛穩定性電子式控制功能（VSF）之激化與順序之關係。 5.5.7.2.4 參考車輛及受模擬車輛配置之物理因子差異應於模擬中進行相對應的修正。 5.5.7.2.5 應檢附模擬器之測試報告。 |  |
| Annex 21 SPECIAL REQUIREMENTS FOR VEHICLES EQUIPPED WITH A VEHICLE STABILITY FUNCTION 1. GENERAL This annex defines the special requirements for vehicles equipped with a vehicle stability function, pursuant to paragraphs 5.2.1.32. and 5.2.2.23. of this Regulation. |  | 6.9車輛穩定性電子式控制功能（VSF）之特殊規定 |  |
| 2. REQUIREMENTS 2.1. Power-driven vehicles 2.1.1. Where a vehicle is equipped with a vehicle stability function as defined in paragraph 2.34. of this Regulation, the following shall apply: |  | 6.9.1 一般規定 6.9.1.1 動力驅動車輛 6.9.1.1.1 配有符合2.20 定義之車輛穩定性電子式控制功能（VSF）之車輛，需符合下述規範： |  |
| In the case of directional control the function shall have the ability to automatically control individually the speed of the left and right wheels on each axle or an axle of each axle group 1/ by selective braking based on the evaluation of actual vehicle behaviour in comparison with a determination of vehicle behaviour demanded by the driver. 2/ |  | 方向性控制之功能，應在評估實際車輛之狀況並比對駕駛之操作要求下（對車輛之其他系統或組件產生額外影響是被允許的。這些系統或組件所造成之額外影響必須遵循其所對應之規範(例如：轉向系統之影響應依循四十七、轉向系統，以用來修正轉向)），藉由選擇性煞車系統來獨立地自動控制各輪軸或輪軸組各輪軸左右輪速之能力（多輪軸係指相鄰兩軸之距離大於兩公尺時，則每一個別輪軸應被視為獨立之輪組）。 |  |
| In the case of roll-over control the function shall have the ability to automatically control the wheel speeds on at least two wheels of each axle or axle group 1/ by selective braking or automatically commanded braking based on the evaluation of actual vehicle behaviour that may lead to vehicle roll-over. 2/ |  | 翻覆性控制之功能，應在評估實際車輛之狀況後，在可能會造成車輛翻覆之狀況下（對車輛之其他系統或組件產生額外影響是被允許的。這些系統或組件所造成之額外影響必須遵循其所對應之規範(例如：轉向系統之影響應依循四十七、轉向系統，以用來修正轉向)），藉由選擇性煞車系統或自動控制煞車系統來自動地控制各輪軸或各輪軸組至少兩輪輪速之能力（多輪軸係指相鄰兩軸之距離大於兩公尺時，則每一個別輪軸應被視為獨立之輪組）。 |  |
| In both cases, the function is not required: (a) When the vehicle speed is below 20 km/h; (b) Until the initial start-up self test and plausibility checks have been completed; (c) When the vehicle is being driven in reverse. |  | 上述項目中，下述情況可不用符合： (1) 當車速低於二十公里/小時。 (2) 在系統完成初期自我測試及可信度檢測之前。 (3) 當車輛為倒車行駛之狀態時。 |  |
| 1/ In the case of multiple axles, where the spread between one axle and its adjacent axle is greater than 2m, each individual axle shall be considered as an independent axle group. 2/ Additional interaction with other vehicle systems or components is allowed. Where these systems or components are subject to special Regulations, such interaction shall comply with the requirements of those Regulations, e.g. interaction with the steering system shall comply with the requirements set out in Regulation No. 79 for corrective steering. |  | 1/多輪軸係指相鄰兩軸之距離大於兩公尺時，則每一個別輪軸應被視為獨立之輪組。 2/允許對車輛之其他系統或部品產生額外影響是被允許的。這些系統或部品所造成之額外影響必須遵循其所對應之規範(例如：轉向系統之影響應依循四十七、轉向系統，以用來修正轉向。 |  |
| 2.1.2. To realise the functionality defined above a vehicle stability function shall include, in addition to selective braking and/or automatically commanded braking, at least the following: (a) The ability to control engine power output. |  | 6.9.1.2車輛穩定性電子式控制功能（VSF）除包含選擇性煞車系統及/或自動控制煞車系統外，仍須最少符合下述功能之一： (1) 控制引擎動力輸出之能力。 |  |
| (b) In the case of directional control: The determination of actual vehicle behaviour from values of yaw rate, lateral acceleration, wheel speeds, and from the driver's control inputs to the braking and steering systems and to the engine. Only onboard generated information shall be used. If these values are not directly measured, the evidence of the appropriate correlation with directly measured values under all driving conditions (e.g. including driving in a tunnel) shall be shown to the technical service at the time of type approval. |  | (2)方向性控制：依據車輛橫擺角速度（Yaw rate）、側向加速度、輪速及駕駛者所控制之煞車、轉向與油門操作，來判斷實際車輛行為。只能使用車輛本身行為所產生之資訊。假使這些數據無法直接量測，須於申請車輛型式認證時向檢測機構提出所有駕駛狀態下其與直接量測值之關聯性(例如：行駛於隧道中)。 |  |
| (c) In the case of roll-over control: The determination of actual vehicle behaviour from values of the vertical force on the tyre(s) (or at least lateral acceleration and wheel speeds) and from the driver's control inputs to the braking system and to the engine. Only on-board generated information shall be used. If these values are not directly measured, the evidence of the appropriate correlation with directly measured values under all driving conditions (e.g. including driving in a tunnel) shall be shown to the technical service at the time of type approval. |  | (3)翻覆性控制：藉由車輪之垂直向受力(或至少藉由側向加速度及輪速)以及駕駛者所控制之煞車與引擎操作來判斷實際車輛行為。僅能採用車輛本身行為所產生之資訊。假使這些數據無法直接量測得，須於申請車輛型式認證時向檢測機構提出所有駕駛狀態下其與直接量測值之關聯性(例如：行駛於隧道中)。 |  |
| (d) In the case of a towing vehicle equipped according to paragraph 5.1.3.1. of this Regulation: The ability to apply the service brakes of the trailer via the respective control line(s) independently of the driver. |  | (4) 配有4.1.4.壓縮空氣煞車之曳引車：能透過各自獨立之控制線路，提供駕駛者操作拖車常用煞車之能力。 |  |
| 2.1.3. The vehicle stability function shall be demonstrated to the Technical Service by dynamic manoeuvres on one vehicle. This may be realized by a comparison of results obtained with the vehicle stability function enabled and disabled for a given load condition. As an alternative to carrying-out dynamic manoeuvres for other vehicles and other load conditions, fitted with the same vehicle stability system, the results from actual vehicle tests or computer simulations may be submitted. The use of the simulator is defined in Appendix 1 to this annex. The specification and validation of the simulator is defined in Appendix 2 to this annex. |  | 6.9.1.3車輛穩定性電子式控制功能（VSF）須由申請者展示實車動態操作給檢測機構來認可。可於同一載重情況下，將車輛穩定性電子式控制功能（VSF）啟用及關閉來比對所獲得之結果。於其他車輛及其他載重情況時執行動態操作之替代方案為使用同一套車輛穩定性電子式控制功能（VSF），實車試驗或電腦模擬之結果皆應可被認同。 模擬裝置之使用應符合6.9.3規定。 模擬裝置之規格及認證應符合6.9.4規定。 |  |
| Until unified test procedures are agreed, the method by which this demonstration is carried out shall be agreed between the vehicle manufacturer and the Technical Service and shall include the critical conditions of directional control and roll-over control as appropriate to the vehicle stability function installed on the vehicle with the method of demonstration and results being appended to the type approval report. This may be carried-out other than at the time of type approval. As a means of demonstrating the vehicle stability function any of the following dynamic manoeuvres shall be used 3/: |  | 在測試程序決定前，此測試方法必須經過車廠及檢測機構之同意，且試驗車輛須裝配車輛穩定性電子式控制功能（VSF）之方向性控制及翻覆性控制，其試驗及試驗結果必須附加至型式認證報告中。 車輛穩定性電子式控制功能（VSF）之動態操作，可使用下述任一之驗證方法（使用檢測機構同意之替代驗證方法時，其結果不能缺少方向性控制或翻覆性控制）： |  |
| Directional Control; Roll-Over Control. Reducing radius test; Steady state circular test. Step steer input test; J-turn. Sine with dwell. J-turn. mu-split single lane change. Double lane change. Reversed steering test or "fish hook" test. Asymmetrical one period sine steer or pulse steer input test. To demonstrate repeatability the vehicle will be subject to a second demonstration using the selected manoeuvre(s). |  | 方向性控制; 翻覆性控制. 縮減半徑試驗; 穩態繞圓試驗. 方向盤轉角階躍試驗; J型轉彎試驗. 正弦定頻試驗. J型轉彎試驗. 不同摩擦係數路面之單車道變換試驗. 雙車道變換試驗. 倒駛轉向試驗或魚鉤試驗. 非對稱一次性正弦轉向試驗或方向盤轉角脈衝試驗. 為了展現其重覆性，須以選定之操作進行第二次展示。 |  |
| 3/ Should the use of any of the above defined manoeuvres not result in loss of directional control or roll-over as appropriate an alternative manoeuvre may be used in agreement with the Technical Service. |  | 3/使用檢測機構同意之替代驗證方法時，其結果不能缺少方向性控制或翻覆性控制。 |  |
| 2.1.4. Interventions of the vehicle stability function shall be indicated to the driver by a specific optical warning signal. The indication shall be present as long as the vehicle stability function is in an intervention mode. The warning signals specified in paragraph 5.2.1.29. of this Regulation shall not be used for this purpose. Interventions of the vehicle stability function used in any learning process to determine the vehicle operational characteristics shall not generate the above signal. The signal shall be visible to the driver, even in daylight, such that the driver can easily verify the satisfactory condition of the signal without leaving the driver's seat. |  | 6.9.1.4車輛穩定性電子式控制功能（VSF）處於介入模式時，需有一特定警示燈訊號告知駕駛者。此警示燈作動之時間，須與車輛穩定性電子式控制功能（VSF）處於介入模式之時間一致。5.1.9.、6.1.16.中所定義之警示燈不得使用於此目的。 車輛穩定性電子式控制功能（VSF）處於介入模式時，若屬於學習程序之判斷車輛操作特徵時，則不該產生上述訊號。 即使在白天，駕駛仍應能清楚可見警示燈訊號，能使駕駛無須離開駕駛座亦能確認警示燈訊號之現況。 |  |
| 2.1.5. A vehicle stability function failure or defect shall be detected and indicated to the driver by the specific optical warning signal referred to in paragraph 5.2.1.29. of this Regulation. The warning signal shall be constant and remain displayed as long as the failure or defect persists and the ignition (start) switch is in the 'on' (run) position. |  | 6.9.1.5 當車輛穩定性電子式控制功能（VSF）失效或故障時，應被偵測出並透過警示燈訊號告知駕駛者，如5.1.9.、6.1.16. 之規定。 當失效或故障之情況持續存在且啟動開關位於「開-ON」時，警告燈訊號應持續顯示且訊號穩定呈現。 |  |
| 2.1.6. In the case of a power-driven vehicle equipped with an electric control line and electrically connected to a trailer with an electric control line the driver shall be warned by a specific optical warning signal whenever the trailer provides the information "VDC Active" via the data communications part of the electric control line. The optical signal defined in paragraph 2.1.4. above may be used for this purpose. |  | 6.9.1.6 備有電力控制線路之動力驅動車輛，且電力聯結至具有電力控制線路之拖車，當拖車透過電力控制線路傳送出一”VDC 作動”之訊號時，應有一特定警示燈訊號告知駕駛者。上述6.9.1.4 所定義之警示燈訊號適用於此目的。 |  |
| 2.2. Trailers 2.2.1. Where a trailer is equipped with a vehicle stability function as defined in paragraph 2.34. of this Regulation, the following shall apply: In the case of directional control the function shall have the ability to automatically control individually the speed of the left and right wheels on each axle or an axle of each axle group 4/ by selective braking based on the evaluation of actual trailer behaviour in comparison with a determination of the relative behaviour of the towing vehicle. 5/ In the case of roll-over control the function shall have the ability to automatically control the wheel speeds on at least two wheels of each axle or axle group 4/ by selective braking or automatically commanded braking based on the evaluation of actual trailer behaviour that may lead to roll-over. 5/ |  | 6.9.2 拖車 6.9.2.1 配備有2.34定義之車輛穩定性電子式控制功能（VSF）車輛，應符合下列規定： 方向性控制之功能，應在評估實際拖車之狀況並比對相對於曳引車之狀況後（對車輛之其他系統或組件產生額外影響是被允許的。這些系統或組件所造成之額外影響必須遵循其所對應之規範(例如：轉向系統之影響應依循四十七、轉向系統，以用來修正轉向)），藉由選擇性煞車系統來獨立地自動控制各輪軸或輪軸組各輪軸左右輪速之能力（多輪軸係指相鄰兩軸之距離大於兩公尺時，則每一個別輪軸應被視為獨立之輪組）。 翻覆性控制之功能，應在評估實際拖車之狀況後，在可能會造成車輛翻覆之狀況下（對車輛之其他系統或組件產生額外影響是被允許的。這些系統或組件所造成之額外影響必須遵循其所對應之規範(例如：轉向系統之影響應依循四十七、轉向系統，以用來修正轉向)），藉由選擇性煞車系統或自動控制煞車系統來自動地控制各輪軸或輪軸組至少兩輪輪速之能力（多輪軸係指相鄰兩軸之距離大於兩公尺時，則每一個別輪軸應被視為獨立之輪組）。 |  |
| 4/ In the case of multiple axles, where the spread between one axle and its adjacent axle is greater than 2 m, each individual axle shall be considered as an independent axle group. 5/ Additional interaction with other vehicle systems or components is allowed. Where these systems or components are subject to special Regulations, such interaction shall comply with the requirements of those Regulations, e.g. interaction with the steering system shall comply with the requirements set out in Regulation No. 79 for corrective steering. |  | 4/多輪軸係指相鄰兩軸之距離大於兩 公尺時，則每一個別輪軸應被視為獨 立之輪組。 5/ /允許對車輛之其他系統或部品產生額外影響是被允許的。這些系統或部品所造成之額外影響必須遵循其所對應之規範(例如：轉向系統之影響應依循四十七、轉向系統，以用來修正轉向。 |  |
| 2.2.2. To realise the functionality defined above a vehicle stability function shall include, in addition to automatically commanded braking and where appropriate selective braking, at least the following: (a) The determination of actual trailer behaviour from values of the vertical force on the tyre(s), or at least lateral acceleration and wheel speeds. Only on-board generated information shall be used. If these values are not directly measured, the evidence of the appropriate correlation with directly measured values under all driving conditions (e.g. including driving in a tunnel) shall be shown to the technical service at the time of type approval. |  | 6.9.2.2 為了實現上述之功能，車輛穩定性電子式控制功能（VSF）必須除了包含選擇性煞車系統及自動控制系統外，至少仍需包含以下功能： (1)藉由車輪之垂直向受力(或至少藉由側向加速度及輪速)來判斷實際車輛行為。只能使用車輛本身行為所產生之資訊。假使這些數據無法直接量測，須於申請車輛型式認證時向檢測機構提出所有駕駛狀態下其與直接量測值之關聯性(例如：行駛於隧道中)。 |  |
| 2.2.3. The vehicle stability function shall be demonstrated to the Technical Service by dynamic manoeuvres on one vehicle. This may be done by a comparison of results obtained with the vehicle stability function enabled and disabled for a given load condition. As an alternative to carrying-out dynamic manoeuvres for other vehicles and other load conditions, fitted with the same vehicle stability system, the results from actual vehicle tests or computer simulations may be submitted. The use of the simulator is defined in Appendix 1 to this Annex. The specification and validation of the simulator is defined in Appendix 2 to this annex. |  | 6.9.2.3車輛穩定性電子式控制功能（VSF）須由申請者展示實車動態操作給檢測機構來認可。可於同一載重情況下，將車輛穩定性電子式控制功能（VSF）啟用及關閉來比對所獲得之結果。於其他車輛及其他載重情況時執行動態操作之替代方案為使用同一套車輛穩定性電子式控制功能（VSF），實車試驗或電腦模擬之結果皆應可被認同。 模擬裝置之使用應符合6.9.3規定。 模擬裝置之規格及認證應符合6.9.4規定。 |  |
| Until unified test procedures are agreed, the method by which this demonstration is carried out shall be agreed between the trailer manufacturer and the Technical Service and shall include the critical conditions of roll-over control and directional control as appropriate to the vehicle stability function installed on the trailer with the method of demonstration and results being appended to the type approval report. This may be carried-out other than at the time of type approval. As a means of demonstrating the vehicle stability function any of the following dynamic manoeuvres shall be used 6/: Directional Control; Roll-Over Control. Reducing radius test; Steady state circular test. Step steer input test; J-turn. Sine with dwell. J-turn. mu-split single lane change. Double lane changev. Reversed steering test or "fish hook" test. Asymmetrical one period sine steer or pulse steer input test. To demonstrate repeatability the vehicle will be subject to a second demonstration using the selected manoeuvre(s). |  | 在測試程序決定前，此測試方法必須經過拖車製造廠及檢測機構之同意，且試驗車輛須裝配車輛穩定性電子式控制功能（VSF）之方向性控制及翻覆性控制，其試驗及試驗結果必須附加至型式認證報告中。 車輛穩定性電子式控制功能（VSF）之動態操作，可使用下述任一之驗證方法（使用檢測機構同意之替代驗證方法時，其結果不能缺少方向性控制或翻覆性控制）： 方向性控制; 翻覆性控制. 縮減半徑試驗; 穩態繞圓試驗. 方向盤轉角階躍試驗; J型轉彎試驗. 正弦定頻試驗. J型轉彎試驗. 不同摩擦係數路面之單車道變換試驗. 雙車道變換試驗. 倒駛轉向試驗或魚鉤試驗. 非對稱一次性正弦轉向試驗或轉向盤轉角脈衝試驗. 為了展現其重覆性，須以選定之操作進行第二次展示。 |  |
| 6/ Should the use of any of the above defined manoeuvres not result in loss of directional control or roll-over as appropriate an alternative manoeuvre may be used in agreement with the technical service. |  | 6/使用檢測機構同意之替代驗證方法時，其結果不能缺少方向性控制或翻覆性控制。 |  |
| 2.2.4. Trailers equipped with an electric control line, when electrically connected to a towing vehicle with an electric control line, shall provide the information "VDC active" via the data communications part of the electric control line when the vehicle stability function is in an intervention mode. Interventions of the vehicle stability function used in any learning process to determine the trailer operational characteristics shall not generate the above information. |  | 6.9.2.4 備有電力控制線路之拖車，且 電力聯結至曳引車時，當車輛穩定性電子式控制功能（VSF）處於介入模式時，電子控制線路之資料通訊系統應提供一”VDC作動”之資訊。當車輛穩定性電子式控制功能（VSF）之介入屬於學習程序之判斷車輛操作特徵時，應不產生上述訊號。 |  |
| 2.2.5. To maximise the performance of trailers that utilise "select-low" such trailers are permitted to change control mode to "select-high" during an intervention of the "Vehicle Stability Function". |  | 6.9.2.5. 為使拖車之性能達到最佳，可於車輛穩定性電子式控制功能（VSF）介入時，對於使用”select-low”之拖車允許變更控制模式至”select-high”。 |  |
| Annex 21 - Appendix 1 USE OF THE DYNAMIC STABILITY SIMULATION The effectiveness of the directional and/or roll-over stability control function of power-driven vehicles and trailers of categories M, N and O, may be determined by computer simulation. |  | 6.9.3 動態穩定性模擬 M、N及O類之動力驅動車輛及拖車，其方向性及/或翻覆性穩定控制能力之效力，可藉由電腦模擬來評估。 |  |
| 1. USE OF THE SIMULATION 1.1. The vehicle stability function shall be demonstrated by the vehicle manufacturer to the Type Approval Authority or Technical Service with the same dynamic manoeuvre(s) as for the practical demonstration in paragraph 2.1.3. or 2.2.3. of Annex 21. |  | 6.9.3.1 模擬之使用 6.9.3.1.1 車輛之穩定功能應由申請者以模擬6.9.1.3 之動態操作展示予檢測機構。 |  |
| 1.2. The simulation shall be a means whereby the vehicle stability performance may be demonstrated with the vehicle stability function enabled or disabled, and in the laden and unladen conditions. |  | 6.9.3.1.2 模擬所使用之方法應為藉由將車輛穩定性電子式控制功能（VSF）開啟或關閉、車輛全負載及無負載之條件下，展示車輛穩定性電子式控制功能（VSF）之性能。 |  |
| 1.3. The simulations shall be carried out with a validated modelling and simulation tool. The verification shall be carried out using the same manoeuvre(s) as defined in paragraph 1.1. above. The method by which the simulation tool is validated is given in Annex 21, Appendix 2. |  | 6.9.3.1.3 模擬必須採用有效模型及模擬軟體，且必須比照6.9.3.1 來執行驗證。 |  |
| Annex 21 - Appendix 2 DYNAMIC STABILITY SIMULATION TOOL AND ITS VALIDATION 1. SPECIFICATION OF THE SIMULATION TOOL 1.1. The simulation method shall take into account the main factors which influence the directional and roll motion of the vehicle. A typical model may include the following vehicle parameters in an explicit or implicit form: (a) Axle/wheel (b) Suspension (c) Tyre (d) Chassis/vehicle body (e) Power train/driveline, if applicable (f) Brake system (g) Pay load |  | 6.9.4 動態穩定性模擬工具及其有效性 6.9.4.1 模擬工具之規格 6.9.4.1.1 模擬方法應考量會影響車輛行進方向與翻滾之主要因子。典型的模型可包含下述車輛參數： (a)車軸/車輪； (b)懸吊； (c)輪胎； (d)底盤/車身； (e)傳動系統/驅動系統(drivr line)(以適用者為對象)； (f)煞車系統； (g)車身負載。 |  |
| 1.2. The Vehicle Stability Function shall be added to the simulation model by means of: a) a subsystem (software model) of the simulation tool, or b) the electronic control box in a hardware-in-the-loop configuration. |  | 6.9.4.1.2車輛穩定性電子式控制功能（VSF）應以下述方式加入模擬模型中： (a)模擬工具之輔助系統(軟體模型)；或 (b)於硬體控制迴圈配置中增加電子控制單元。 |  |
| 1.3. In the case of a trailer, the simulation shall be carried out with the trailer coupled to a representative towing vehicle. |  | 6.9.4.1.3 拖車應聯結至具有代表性之曳引車狀態下執行模擬。 |  |
| 1.4. Vehicle loading condition 1.4.1. The simulator shall be able to take into account the laden and unladen conditions. 1.4.2. The load shall be considered to be a fixed load with properties (mass, mass distribution and maximum recommended height of the centre of gravity) specified by the manufacturer. |  | 6.9.4.1.4 車輛負載條件 6.9.4.1.4.1 模擬裝置必須能考量到全負載及無負載狀態。 6.9.4.1.4.2 負載必須是由車廠宣告之固定負載及裝載特性(重量、重量分配及建議之最大重心高度)。 |  |
| 2. VALIDATION OF THE SIMULATION TOOL 2.1. The validity of the applied modelling and simulation tool shall be verified by means of comparisons with a practical vehicle test(s). The test(s) utilised for the validation shall be those which, without control action, would result in loss of directional control (understeer and over-steer) or roll-over control as appropriate to the functionality of the stability control function installed on a representative vehicle. |  | 6.9.4.2 模擬軟體之驗證 6.9.4.2.1. 應藉由比較車輛實際測試結果來驗證所使用模型及模擬工具之有效性，應以足以展現代表車所配備之車輛穩定性電子式控制功能（VSF）之測試方法進行驗證，其應為在未給予控制操作時會導致失去方向性控制（轉向不足及過度轉向）或翻覆性控制之測試方法。 |  |
| During the test(s) the following motion variables, as appropriate, shall be recorded or calculated in accordance with ISO 15037 Part 1:2005: General conditions for passenger cars or Part 2:2002: General conditions for heavy vehicles and buses (depending on the vehicle category): (a) yaw velocity; (b) lateral acceleration; (c) wheel load or wheel lift; (d) forward velocity; (e) driver input. |  | 於測試期間，下述運動參數應視實際適當狀況予以記錄或依照ISO 15037 Part1:2005(小客車之一般狀態)或Part2 :2002(大型車輛之一般狀態)計算(視車輛種類而定)。 (1) 橫擺角速度(yaw rate) (2) 側向加速度 (3) 車輪負載或車輪離地 (4) 前進速度 (5) 駕駛者之操作 |  |
| 2.2. The objective is to show that the simulated vehicle behaviour and operation of the vehicle stability function is comparable with that seen in practical vehicle tests. |  | 6.9.4.2.2 以呈現被模擬車輛之行為及車輛穩定性電子式控制功能（VSF）之操作與實際車輛測試結果可相當為目標。 |  |
| 2.3. The simulator shall be deemed to be validated when its output is comparable to the practical test results produced by a given vehicle type during the selected manoeuvre(s) from those defined with paragraph 2.1.3. or 2.2.3. of Annex 21, as appropriate. In the case of the steady state circular test the under-steer gradient shall be the means of making the comparison. In the case of a dynamic manoeuvre, the relationship of activation and sequence of the vehicle stability function in the simulation and in the practical vehicle test shall be the means of making the comparison. |  | 6.9.4.2.3 當模擬裝置之輸出結果相當於6.9.1.3 實車試驗之結果，則模擬裝置將視為有效。 於穩態繞圓試驗時，應以轉向不足程度（gradient）作為比較之方法。 於動態試驗，應以模擬及實車試驗中，車輛穩定性電子式控制功能（VSF）之啟動及連續作動作為比較之方法。 |  |
| 2.4. The physical parameters that are different between the reference vehicle and simulated vehicle configurations shall be modified accordingly in the simulation. |  | 6.9.4.2.4 參考車輛及受模擬車輛配置之物理因子差異應於模擬中進行相對應的修正。 |  |
| 2.5. A simulator test report shall be produced, a model of which is defined in Appendix 3 of this annex, and a copy attached to the vehicle approval report. |  | 6.9.4.2.5 應檢附模擬裝置之測試報告。 |  |

基準實施日期列表

版本日期：99/09/6

| 項目 |  | 實施日期 | 適用範圍 |
| --- | --- | --- | --- |
| 二、車輛規格規定 | 座椅應裝設三點式安全帶 | 一０四年一月一日起 | 新型式M1及總重量小於三‧五公噸之新型式M2類車輛之座椅應裝設三點式安全帶。新型式Ｎ、M3及總重量大於三‧五公噸之新型式M2類車輛之前排兩側座椅應裝設三點式安全帶，其餘座椅應裝設三點式或二點式安全帶 |
|  |  | 一０八年一月一日起 | 各型式M1及總重量小於三‧五公噸之各型式M2類車輛之座椅應裝設三點式安全帶。各型式Ｎ、M3及總重量大於三‧五公噸之各型式M2類車輛之前排兩側座椅應裝設三點式安全帶，其餘座椅應裝設三點式或二點式安全帶 |
|  | 駕駛座配備安全帶提醒裝置 | 一０二年一月一日起 | 新型式M1類車輛駕駛座及中華民國一０四年一月一日起，各型式M1類車輛駕駛座應配備安全帶提醒裝置。車輛製造廠在其他種類車輛的駕駛座配備安全帶提醒系統，亦可依此規定申請認證 |
|  | 大客車設有自行車停放區（草案） | 公告後實施 | 甲類、乙類、丙類及丁類大客車，設有自行車停放區者須符合本項規定。 |
| 三之一、車輛燈光與標誌檢驗規定：自九十五年七月一日起實施 |  | 九十五年七月一日起 | 新型式之M2、M3、N2、N3及Ｏ類車輛應符合本項4.、6.及7.之規定 |
|  |  | 九十七年七月一日起 | 各型式之M2、M3、N2、N3及Ｏ類車輛，應符合本項4.、6.及7.之規定，惟其後霧燈得為選配並可就4.5.1或4.5.2之規定擇一符合 |
|  |  | 九十七年七月一日起 | 新型式之M2、M3、N2、N3及Ｏ類車輛，其後霧燈應符合本項4.5.1之規定 |
|  |  | 九十九年七月一日起 | 各型式之M2、M3、N2、N3及Ｏ類車輛，其後霧燈應符合本項4.5.1之規定 |
|  |  | 九十五年七月一日起 | 新型式之L1及L3類車輛，其氣體放電式頭燈，應符合本項5.之規定 |
|  |  | 九十七年七月一日起 | 各型式之L1及L3類車輛，其氣體放電式頭燈，應符合本項5.之規定 |
|  |  | 九十七年一月一日起 | 新型式之M1及N1類車輛，其車輛燈光與標誌，應符合本項4.、6.及7.之規定，惟其後霧燈僅能適用4.5.1之規定 |
|  |  | 九十九年一月一日起 | 各型式之M1及N1類車輛，其車輛燈光與標誌，應符合本項4.、6.及7.之規定，惟其後霧燈僅能適用4.5.1之規定 |
|  |  | 九十八年一月一日起 | 新型式之L1及L3類車輛，其車輛燈光與標誌，應符合本項5.至7.之規定 |
|  |  | 一００年一月一日起 | 各型式之L1及L3類車輛，其車輛燈光與標誌，應符合本項5.至7.之規定 |
|  |  | 九十八年十二月二十五日起 | 同一申請者同一年度同型式規格車輛，申請少量車型安全審驗且總數未逾二十輛者；或同一申請者同一年度同型式規格車輛，申請逐車少量車型安全審驗且總數未逾二十輛者，得免符合4.2.5.2水平投射之規定。 |
|  |  | 九十九年四月十三日起 | O3及O4類車輛其車身側方及後方帶狀反光識別材料應符合本項6.14.2、6.14.3.1之規定。 |
| 三之二、車輛燈光與標誌檢驗規定：自一００年一月一日起實施 |  | 一００年一月一日起 | 新型式之M、N及Ｏ類車輛，其車輛燈光與標誌，應符合本項4.及6.至8.之規定 |
|  |  | 一０二年一月一日起 | 各型式之M、N及Ｏ類車輛，已符合本基準項次「三之一」之規定且其車輛燈光與標誌配備適路性前方照明系統及/或緊急煞車訊號者，另應符合本項之適路性前方照明系統及/或緊急煞車訊號之相關規定 |
|  |  | 一００年一月一日起 | 新型式之L1及L3類車輛，其車輛燈光與標誌，應符合本項5.至8.之規定 |
|  |  | 九十八年十二月二十五日起 | 同一申請者同一年度同型式規格車輛，申請少量車型安全審驗且總數未逾二十輛者；或同一申請者同一年度同型式規格車輛，申請逐車少量車型安全審驗且總數未逾二十輛者，得免符合4.2.5.2水平投射及/或6.16適路性前方照明系統（AFS）之規定。 |
|  |  | 九十九年四月十三日起 | O3及O4類車輛其車身側方及後方帶狀反光識別材料應符合本項6.14.2、6.14.3.1之規定。 |
| 四、靜態煞車 |  | 九十九年一月一日起 | 總重量不超過七五０公斤之拖車，得免裝設煞車系統。 |
| 七、左右兩側防止捲入裝置與後方安全防護裝置(或保險槓) |  | 公告日起 | 除曳引車以外之N2及N3類車輛、O2、O3、O4類車輛 |
|  |  | 九十三年一月一日起 | 新登記檢驗領照之曳引車 |
| 七之ㄧ、左右兩側防止捲入裝置與後方安全防護裝置(或保險槓)（草案） |  | ○年○月○日起 | 中華民國○○年○月○日起，新型式之N2、N3、O2、O3、O4類車輛及中華民國○○年○月○日起，各型式之N2、N3、O2、O3、O4類車輛，應符合本項規定。 |
| 八、汽車傾斜穩定度規定 |  | 八十九年一月一日起 | 三‧五公尺以上汽車，其傾斜穩定度，應符合下列規定：（2.1~2.2） |
|  |  | 九十六年七月一日起 | 車高三‧四公尺以上之新型式大客車，其傾斜穩定度，應符合下列規定：（3.1~3.2） |
|  |  | 九十七年一月一日起 | 車高三‧四公尺以上之各型式大客車，其傾斜穩定度，應符合下列規定：（3.1~3.2） |
| 九、喇叭音量 |  | 九十年七月一日起 | 汽車、機器腳踏車 |
| 九之一、聲音警告裝置(喇叭)安裝規定：自九十五年七月一日起實施 |  | 九十五年七月一日起 | 新型式之Ｍ及Ｎ類車輛 |
|  |  | 九十七年七月一日起 | 各型式之Ｍ及Ｎ類車輛 |
|  |  | 九十六年一月一日起 | 各型式之L1及L3類車輛 |
|  |  | 九十八年一月一日起 | 各型式之L1及L3類車輛，其聲音警告裝置之安裝，應符合本項規定；且應使用符合本基準規定之聲音警告裝置 |
| 十、載重計安裝規定 |  | 九十年七月一日起 | 裝載砂石、土方之傾卸式半拖車及二十噸以上傾卸式大貨車等車輛 |
| 十一、轉彎及倒車警報裝置安裝規定 |  | 九十年七月一日起 | 裝載砂石、土方之傾卸式大貨車及傾卸式半拖車等車輛 |
| 十二、機器腳踏車排氣系統隔熱防護裝置 |  | 九十一年一月一日起 | 機器腳踏車 |
| 十三、機器腳踏車腳架穩定性與耐久性規定 |  | 九十三年一月一日起 | L1及L3類車輛 |
| 十四、機器腳踏車客座扶手規定 |  | 九十四年一月一日起 | 1L1及L3類車輛 |
| 十五、載重計 |  | 九十年七月一日起 | 裝載砂石、土方之傾卸式半拖車及二十噸以上傾卸式大貨車等車輛 |
| 十六、行車紀錄器 |  | 公告日起 | 總聯結重量及總重量在二十公噸以上之M及N類車輛 |
|  |  | 九十年一月一日起 | 八公噸以上未滿二十公噸之M及N類車輛 |
|  |  | 九十六年七月一日起 | 新型式之八公噸以下大客車 |
|  |  | 九十七年一月一日起 | 各型式之八公噸以下大客車 |
| 十八、小型汽車置放架之靜態強度 |  | 九十八年十二月二十五日起 | 於適用型式及其範圍認定原則增訂「安裝位置」相同之規定。 |
| 十九、車輛內裝材料難燃性能要求 |  | 九十一年一月一日起 | 幼童專用車、校車、大客車、小客車及小客貨兩用車之新車型 |
|  |  | 九十三年一月一日起 | 幼童專用車、校車、大客車、小客車及小客貨兩用車之各車型 |
| 二十之一、反光識別材料：自九十五年七月一日起實施 |  | 九十五年七月一日起 | 使用於幼童專用車及校車車身左右兩側與後方車身標示之倒三角形黃色部分之新型式反光識別材料 |
|  |  | 九十七年七月一日起 | 使用於幼童專用車及校車車身左右兩側與後方車身標示之倒三角形黃色部分之各型式反光識別材料 |
|  |  | 九十五年七月一日起 | 使用於除前述車輛（幼童專用車及校車）外之M、N及O類車輛之新型式反光識別材料 |
|  |  | 九十七年七月一日起 | 使用於除前述車輛（幼童專用車及校車）外之M、N及O類車輛之各型式反光識別材料 |
| 二十之二、反光識別材料：自一００年一月一日起實施 |  | 一００年一月一日起 | 使用於幼童專用車及校車車身左右兩側與後方車身標示之倒三角形黃色部分之新型式反光識別材料 |
|  |  | 一０二年一月一日起 | 使用於幼童專用車及校車車身左右兩側與後方車身標示之倒三角形黃色部分之已符合本基準項次「二十之一」規定之各型式反光識別材料，另應符合本項7.7及7.8之規定 |
|  |  | 一００年一月一日起 | 使用於除前述車輛（幼童專用車及校車）外之M、N及O類車輛之新型式反光識別材料 |
|  |  | 一０二年一月一日起 | 使用於除前述車輛外之M、N及O類車輛之各型式反光識別材料已符合本基準項次「二十之一」之規定者，另應符合本項7.7及7.8之規定 |
| 二十一、聲音警告裝置(喇叭) |  | 九十五年七月一日起 | 使用於Ｍ及Ｎ類車輛之新型式聲音警告裝置 |
|  |  | 九十七年七月一日起 | 使用於Ｍ及Ｎ類車輛之各型式聲音警告裝置 |
|  |  | 九十六年一月一日起 | 使用於L3類車輛之新型式聲音警告裝置 |
|  |  | 九十八年一月一日起 | 使用於L3類車輛之各型式聲音警告裝置 |
| 二十二、速率計 |  | 九十五年七月一日起 | 新型式之M1、N1、L1及L3類車輛 |
|  |  | 九十七年七月一日起 | 各型式之M1、N1、L1及L3類車輛 |
|  |  | 九十七年一月一日起 | 新型式之M2、M3及N2、N3類車輛 |
|  |  | 九十九年一月一日起 | 各型式之M2、M3及N2、N3類車輛 |
| 二十三、間接視野裝置安裝規定 |  | 九十五年七月一日起 | 新型式之L1和L3類車輛 |
|  |  | 九十七年七月一日起 | 各型式之L1和L3類車輛，其間接視野裝置(照後鏡)安裝應符合本項規定，且應使用符合本基準規定之間接視野裝置(照後鏡) |
|  |  | 一００年一月一日起 | 新型式之Ｍ類和Ｎ類車輛 |
|  |  | 一０二年一月一日起 | 各型式之Ｍ類和Ｎ類車輛，其間接視野裝置安裝應符合本項規定，且應使用符合本基準規定之間接視野裝置 |
| 二十四、機器腳踏車控制器標誌 |  | 九十五年七月一日起 | 新型式之L1及L3類車輛 |
|  |  | 九十七年七月一日起 | 各型式之L1及L3類車輛 |
| 二十五、安全玻璃 |  | 九十五年七月一日起 | 使用於M2及M3類車輛乘室區之新型式安全玻璃(儀表板除外) |
|  |  | 九十七年七月一日起 | 使用於M2及M3類車輛乘室區之各型式安全玻璃(儀表板除外) |
|  |  | 九十五年七月一日起 | 使用於M1及N類車輛之新型式前擋風玻璃，應符合本項規定中「厚度測定」、「耐衝擊性試驗」、「耐貫穿性試驗」、「人頭模型衝擊試驗」及「可見光透過率試驗」之規定 |
|  |  | 九十七年七月一日起 | 使用於M1及N類車輛之各型式前擋風玻璃，應符合本項規定中「厚度測定」、「耐衝擊性試驗」、「耐貫穿性試驗」、「人頭模型衝擊試驗」及「可見光透過率試驗」之規定 |
|  |  | 九十七年一月一日起 | 使用於M1及N類車輛乘室區之新型式安全玻璃(儀表板除外) |
|  |  | 九十九年一月一日起 | 使用於M1及N類車輛乘室區之各型式安全玻璃(儀表板除外) |
|  |  | 九十八年十二月二十五日起 | 九十九年一月一日前，前擋風玻璃不得低於百分之七五，前擋風玻璃以外之玻璃窗不得低於百分之七０。自九十九年一月一日起，前擋風玻璃或前擋風玻璃以外之玻璃窗不得低於百分之七０。 |
| 二十五之一、安全玻璃：自一００年一月一日起實施 |  | 一００年一月一日起 | 使用於M及N類車輛乘室區之新型式安全玻璃(儀表板除外) |
| 二十六、安全帶 |  | 九十五年七月一日起 | 使用於Ｍ及Ｎ類車輛之新型式安全帶 |
|  |  | 九十七年七月一日起 | 使用於Ｍ及Ｎ類車輛之各型式安全帶 |
| 二十七、間接視野裝置 |  | 九十五年七月一日起 | 使用於L1及L3類車輛之新型式間接視野裝置(照後鏡) |
|  |  | 九十七年七月一日起 | 使用於L1及L3類車輛之各型式間接視野裝置(照後鏡)，應符合本項規定 |
|  |  | 一００年一月一日起 | 使用於M及N類車輛之新型式間接視野裝置 |
|  |  | 一０二年一月一日起 | 使用於M及N類車輛之各型式間接視野裝置，應符合本項規定 |
| 二十八、輪胎 |  | 九十五年七月一日起 | 使用於M、N及O類車輛之新型式輪胎 |
|  |  | 九十七年七月一日起 | 使用於M、N及O類車輛之各型式輪胎 |
| 二十八之一、輪胎：自一０二年一月一日起實施 |  | 一０二年一月一日起 | 使用於M、N、O及L類車輛之新型式輪胎應符合本項規定 |
|  |  | 一０四年一月一日起 | 使用於M1、O1及O2類車輛之各型式失壓續跑輪胎及速度超過三００公里/小時之輪胎，應符合本項規定 |
|  |  | 一０四年一月一日起 | 使用於L類車輛之各型式輪胎，應符合本項規定 |
| 二十九、燈泡 |  | 九十五年七月一日起 | M、N及O類車輛其車輛型式安全審驗相關燈具所使用之新型式燈泡 |
|  |  | 九十七年七月一日起 | M、N及O類車輛其車輛型式安全審驗相關燈具所使用之各型式燈泡 |
|  |  | 九十八年一月一日起 | L1及L3類車輛其車輛型式安全審驗相關燈具所使用之新型式燈泡 |
|  |  | 一００年一月一日起 | L1及L3類車輛其車輛型式安全審驗相關燈具所使用之各型式燈泡 |
| 三十、氣體放電式頭燈 |  | 九十五年七月一日起 | 使用於M、N、L1及L3類車輛之新型式氣體放電式頭燈 |
|  |  | 九十七年七月一日起 | 使用於M、N、L1及L3類車輛之各型式氣體放電式頭燈 |
| 三十之一、氣體放電式頭燈：自一００年一月一日起實施 |  | 一００年一月一日起 | 使用於M、N及L3類車輛之新型式氣體放電式頭燈 |
|  |  | 一０二年一月一日起 | 使用於M、N及L3類車輛之各型式氣體放電式頭燈，已符合本基準項次「三十」之規定且裝設額外光源及/或具備轉彎光型者，另應符合本項之裝設額外光源及/或具備轉彎光型之相關規定 |
| 三十一、方向燈 |  | 九十五年七月一日起 | 使用於M、N及O類車輛之新型式方向燈 |
|  |  | 九十七年七月一日起 | 使用於M、N及O類車輛之各型式方向燈，應符合本項規定，且應使用符合本基準中「燈泡」規定之燈泡 |
|  |  | 九十八年一月一日起 | 使用於L1及L3類車輛之新型式方向燈 |
|  |  | 一００年一月一日起 | 使用於L1及L3類車輛之各型式方向燈，應符合本項規定，且應使用符合本基準中「燈泡」規定之燈泡 |
| 三十二、前霧燈 |  | 九十五年七月一日起 | 使用於M及N類車輛之新型式前霧燈 |
|  |  | 九十七年七月一日起 | 使用於M及N類車輛之各型式前霧燈，應符合本項規定，且應使用符合本基準中「燈泡」規定之燈泡 |
|  |  | 九十六年一月一日起 | 使用於L3類車輛之新型式前霧燈 |
|  |  | 九十八年一月一日起 | 使用於L3類車輛之各型式前霧燈，應符合本項規定，且應使用符合本基準中「燈泡」規定之燈泡 |
| 三十二之一、前霧燈：自一０二年一月一日起實施 |  | 一０二年一月一日起 | 新型式之L、M、N類車輛 |
|  |  | 一０六年一月一日起 | 各型式之L、M、N類車輛 |
| 三十三、倒車燈 |  | 九十五年七月一日起 | 使用於M、N及O類車輛之新型式倒車燈 |
|  |  | 九十七年七月一日起 | 使用於M、N及O類車輛之各型式倒車燈，其倒車燈應符合本項規定，且應使用符合本基準中「燈泡」規定之燈泡 |
| 三十四、車寬燈(前(側)位置燈) |  | 九十五年七月一日起 | 使用於M、N及O類車輛之新型式車寬燈(前(側)位置燈) |
|  |  | 九十七年七月一日起 | 使用於M、N及O類車輛之各型式車寬燈(前(側)位置燈)，應符合本項規定，且應使用符合本基準中「燈泡」規定之燈泡 |
|  |  | 九十八年一月一日起 | 使用於L1及L3類車輛之新型式車寬燈(前(側)位置燈) |
|  |  | 一００年一月一日起 | 使用於L1及L3類車輛之各型式車寬燈(前(側)位置燈)，應符合本項規定，且應使用符合本基準中「燈泡」規定之燈泡 |
| 三十五、尾燈(後(側)位置燈) |  | 九十五年七月一日起 | 使用於M、N及O類車輛之新型式尾燈(後(側)位置燈) |
|  |  | 九十七年七月一日起 | 使用於M、N及O類車輛之各型式尾燈(後(側)位置燈)，應符合本項規定，且應使用符合本基準中「燈泡」規定之燈泡 |
|  |  | 九十八年一月一日起 | 使用於L1及L3類車輛之新型式尾燈(後(側)位置燈) |
|  |  | 一００年一月一日起 | 使用於L1及L3類車輛之各型式尾燈(後(側)位置燈)，應符合本項規定，且應使用符合本基準中「燈泡」規定之燈泡 |
| 三十六、停車燈 |  | 九十五年七月一日起 | 使用於M、N及O類車輛之新型式停車燈 |
|  |  | 九十七年七月一日起 | 使用於M、N及O類車輛之各型式停車燈，應符合本項規定，且應使用符合本基準中「燈泡」規定之燈泡 |
| 三十七、煞車燈 |  | 九十五年七月一日起 | 使用於M、N及O類車輛之新型式煞車燈 |
|  |  | 九十七年七月一日起 | 使用於M、N及O類車輛之各型式煞車燈，應符合本項規定，且應使用符合本基準中「燈泡」規定之燈泡 |
|  |  | 九十八年一月一日起 | 使用於L1及L3類車輛之新型式煞車燈 |
|  |  | 一００年一月一日起 | 使用於L1及L3類車輛之各型式煞車燈，應符合本項規定，且應使用符合本基準中「燈泡」規定之燈泡 |
| 三十八、第三煞車燈 |  | 九十五年七月一日起 | 使用於M、N及O類車輛之新型式第三煞車燈 |
|  |  | 九十七年七月一日起 | 使用於M、N及O類車輛之各型式第三煞車燈，應符合本項規定，且應使用符合本基準中「燈泡」規定之燈泡 |
| 三十九、輪廓邊界標識燈 |  | 九十五年七月一日起 | 使用於M、N及O類車輛之新型式輪廓邊界標識燈 |
|  |  | 九十七年七月一日起 | 使用於M、N及O類車輛之各型式輪廓邊界標識燈，應符合本項規定，且應使用符合本基準中「燈泡」規定之燈泡 |
| 四十、側方標識燈 |  | 九十五年七月一日起 | 使用於M、N及O類車輛之新型式側方標識燈 |
|  |  | 九十七年七月一日起 | 使用於M、N及O類車輛之各型式側方標識燈，應符合本項規定，且應使用符合本基準中「燈泡」規定之燈泡 |
| 四十之一、側方標識燈：自一００年一月一日起實施 |  | 一００年一月一日起 | 使用於M、N及O類車輛之新型式側方標識燈，應符合本項規定，且應使用符合本基準中「燈泡」規定之燈泡 |
|  |  | 一０二年一月一日起 | 使用於M、N及O類車輛之各型式側方標識燈，已符合本基準項次「四十」之規定且發光顏色為紅色者，另應符合本項之紅色側方標識燈光度與照射角度規定 |
| 四十一、反光標誌(反光片) |  | 九十五年七月一日起 | 使用於M、N及O類車輛之新型式反光標誌 |
|  |  | 九十七年七月一日起 | 使用於M、N及O類車輛之各型式反光標誌 |
|  |  | 九十八年一月一日起 | 使用於L1及L3類車輛之新型式反光標誌 |
|  |  | 一００年一月一日起 | 使用於L1及L3類車輛之各型式反光標誌 |
| 四十一之一、反光標誌(反光片)：自一００年一月一日起實施 |  | 一００年一月一日起 | 使用於M、N、O、L1及L3類車輛之新型式反光標誌 |
| 四十二、動態煞車：自九十六年一月一日起實施 |  | 九十六年一月一日起 | 新型式之L1及L3類車輛 |
|  |  | 九十八年一月一日起 | 各型式之L1及L3類車輛 |
|  |  | 九十七年一月一日起 | 新型式之Ｍ1、Ｎ類及Ｏ類車輛 |
|  |  | 九十九年一月一日起 | 各型式之Ｍ1、Ｎ類及Ｏ類車輛 |
|  |  | 九十七年一月一日起 | 新型式之Ｍ2及M3類車輛 |
|  |  | 九十八年一月一日起 | 各型式之Ｍ2及M3類車輛 |
|  |  | 九十八年十二月二十五日起 | 同一申請者同一年度同型式規格之M1或N1類車輛申請少量車型安全審驗且總數未逾二十輛或機關、學校進口自行使用之N2、N3類及丙、丁類大客車，得免符合本項「動態煞車」規定中第二煞車系統性能及能量儲存裝置測試。 |
| 四十二之一、動態煞車：自一００年一月一日起實施(修正草案) |  | 一００年一月一日起 | 新型式之Ｍ、Ｎ及Ｏ類非電動車輛。 |
|  |  | 一００年七月一日起 | 新型式之Ｍ、Ｎ及Ｏ類電動車輛 |
|  |  | 一０二年七月一日起 | 各型式之Ｍ、Ｎ及Ｏ類電動車輛，配備電力再生煞車系統者 |
|  |  | 一０二年一月一日起 | 各型式之Ｍ、Ｎ及Ｏ類車輛已符合本基準項次「四十二」之規定，配備自動煞車或選擇性煞車者，另應符合本項5.1.12之規定；配備緊急煞車信號功能者，另應符合本項5.1.13及6.1.21之規定 |
|  |  | 一００年一月一日起 | 同一申請者同一年度同型式規格之M1或N1類車輛申請少量車型安全審驗且總數未逾二十輛或機關、學校進口自行使用之N2、N3類及丙、丁類大客車，得免符合本項「動態煞車」規定中第二煞車系統性能及能量儲存裝置測試。 |
| 四十二之二、動態煞車：自一０二年一月一日起實施 |  | 一０二年一月一日起 | 新型式之Ｍ、Ｎ、Ｏ、L1及L3類車輛及中華民國一０六年一月一日起，各型式之L1及L3類車輛，其動態煞車，應符合本項規定。符合本基準項次「四十二之一」規定之Ｍ、Ｎ及Ｏ類車輛，亦視同符合本項規定 |
| 四十二之三、動態煞車：自○年○月○日起實施(草案) |  | ○年○月○日起 | 中華民國○年○月○日起，新型式之Ｍ1、Ｎ1、Ｏ1、O2、L1及L3類車輛，其動態煞車，應符合本項規定。 新型式之M1及選擇符合5.規範之N1類車輛，若配備有電子穩定控制系統，則應符合5.5之規定。 |
|  |  | ○年○月○日起 | 車重大於一七三五公斤之新型式M1及N1類車輛，如具備包含翻覆性控制及方向性控制之車輛穩定性電子式控制功能（VSF）者，應符合6.9車輛穩定性電子式控制功能（VSF）之特殊規定。 中華民國○年○月○日起，新型式之M2、M3、N2、N3及O3、O4類車輛，其動態煞車，應符合本項規定。 M2、M3、N2及N3類車輛且其輪軸組不超過三軸者及O3及O4類車輛且其輪軸組不超過三軸並配備氣壓式懸吊者，應符合6.9車輛穩定性電子式控制功能（VSF）之特殊規定。 |
| 四十三、防鎖死煞車系統 |  | 九十七年一月一日起 | 若配備防鎖死煞車系統之新型式之M1及N1車輛 |
|  |  | 九十九年一月一日起 | 若配備防鎖死煞車系統之各型式之M1及N1車輛 |
|  |  | 九十八年一月一日起 | 若配備防鎖死煞車系統之新型式之L1及L3車輛 |
|  |  | 一００年一月一日起 | 若配備防鎖死煞車系統之各型式之L1及L3車輛 |
|  |  | 一００年一月一日起 | 若配備防鎖死煞車系統之超過四軸之新型式M2、M3、N2及N3車輛 |
|  |  | 一０二年一月一日起 | 若配備防鎖死煞車系統之超過四軸之各型式M2、M3、N2及N3車輛 |
|  |  | 一００年一月一日起 | 新型式之O3、O4類車輛和不超過四軸之M2、M3、N2、N3類車輛，應配備防鎖死煞車系統 |
|  |  | 一０二年一月一日起 | 各型式之O3、O4類車輛和不超過四軸之M2、M3、N2、N3類車輛，應配備防鎖死煞車系統 |
|  |  | 九十八年十二月二十五日起 | 同一申請者同一年度同型式規格之M1或N1類車輛，申請少量車型安全審驗且總數未逾二十輛或機關、學校進口自行使用之N2、N3類及丙、丁類大客車，得免符合本項「防鎖死煞車系統」規定中能量消耗試驗及抓地力利用率試驗測試。 |
| 四十三之一、防鎖死煞車系統：自一０二年一月一日起實施 |  | 一０二年一月一日起 | 新型式之O3、O4類車輛和不超過四軸之M2、M3、N2、N3類車輛，應配備防鎖死煞車系統 |
|  |  | 一０四年一月一日起 | 各型式之O4類車輛，已符合本基準項次「四十三」規定者，另應符合本項4.2之規定 |
|  |  | 一０二年一月一日起 | 若配備防鎖死煞車系統之新型式之M1、N1、L1、L3車輛及超過四軸之M2、M3、N2及N3車輛 |
|  |  | 一０四年一月一日起 | 各型式超過四軸之M2、M3、N2及N3車輛，已符合本基準項次「四十三」規定且配備整合式持久煞車系統者，另應符合本項5.1.6之規定 |
|  |  | 一０六年一月一日起 | 各型式之L1及L3類車輛，若配備防鎖死煞車系統，則應符合本項規定。 |
|  |  | 一０二年一月一日起 | 同一申請者同一年度同型式規格之M1或N1類車輛，申請少量車型安全審驗且總數未逾二十輛或機關、學校進口自行使用之N2、N3類及丙、丁類大客車，得免符合本項「防鎖死煞車系統」規定中能量消耗試驗及抓地力利用率試驗測試。 |
| 四十四、轉向控制系駕駛人碰撞保護 |  | 九十七年一月一日起 | 總重量小於一‧五公噸之新型式M1和N1類車輛 |
|  |  | 九十九年一月一日起 | 總重量小於一‧五公噸之各型式M1和N1類車輛 |
| 四十四之一、轉向控制系駕駛人碰撞保護：自一００年一月一日起實施(修正草案) |  | 一００年一月一日起 | 新型式非電動M1和總重量小於一‧五公噸之新型式N1非電動類車輛 |
|  |  | 一０二年一月一日起 | 總重量大於一‧五公噸之各型式M1類非電動車輛 |
|  |  | 一０一年一月一日起 | 新型式M1類電動車輛和總重量小於一‧五公噸之新型式N1類電動車輛 |
|  |  | 一０三年一月一日起 | 各型式M1類電動車輛和總重量小於一‧五公噸之各型式N1類車輛 |
| 四十五、側方碰撞乘員保護 |  | 九十七年一月一日起 | 座椅Ｒ點距地高度小於七００公釐之新型式M1及N1類車輛 |
|  |  | 九十九年一月一日起 | 座椅Ｒ點距地高度小於七００公釐之各型式M1及N1類車輛 |
| 四十五之一、側方碰撞乘員保護：自一０三年一月一日起實施(草案) |  | 一０三年一月一日起 | 座椅Ｒ點距地高度小於七００公釐M1及N1類車輛 |
| 四十六、前方碰撞乘員保護 |  | 九十七年一月一日起 | 總重量小於二‧五公噸之新型式M1類車輛 |
|  |  | 九十九年一月一日起 | 總重量小於二‧五公噸之各型式M1類車輛 |
| 四十六之一、前方碰撞乘員保護：自一０三年一月一日起實施(草案) |  | 一０三年一月一日起 | 總重量小於二‧五公噸M1類車輛 |
| 四十七、轉向系統 |  | 九十七年一月一日起 | 新型式之Ｍ、Ｎ、及Ｏ類車輛 |
|  |  | 九十九年一月一日起 | 各型式之Ｍ、Ｎ、及Ｏ類車輛 |
| 四十八、安全帶固定裝置 |  | 九十七年一月一日起 | 新型式之Ｍ及Ｎ類車輛 |
|  |  | 九十九年一月一日起 | 各型式之Ｍ及Ｎ類車輛 |
|  |  | 九十七年一月一日起 | 安全帶固定裝置之適用型式及其範圍認定原則，應符合以下規定： 2.1.1 車種代號相同 2.1.2廠牌及車輛型式系列相同 2.1.3 底盤車廠牌相同 2.1.4 底盤車製造廠宣告之底盤車型式系列相同 2.1.5 若以底盤車代替完成車執行本項全部或部分檢測時，其適用型式及其範圍認定原則： 2.1.5.1 底盤車廠牌相同 2.1.5.2 底盤車製造廠宣告之底盤車型式系列相同 |
|  |  | 九十八年一月一日起 | 安全帶固定裝置之適用型式及其範圍認定原則，應符合以下規定： 2.2.1廠牌相同 2.2.2固定點數量相同 2.2.3固定裝置結構、尺寸及材質相同 2.2.4與固定裝置各固定點接合之固定方式、結構(含接合固定之鈑件厚度)及材質相同 |
| 四十八之一、安全帶固定裝置：自一０四年一月一日起實施 |  | 一０四年一月一日起 | 新型式之M及N類車輛 |
| 四十九、座椅強度 |  | 九十七年一月一日起 | 使用於M及N類車輛之新型式座椅 |
|  |  | 九十九年一月一日起 | 使用於M及N類車輛之各型式座椅 |
|  |  | 九十八年十二月二十五日起 | 本項規定不適用於折疊式、側向式、後向式座椅及幼童專用車之幼童座椅。 |
| 五十、頭枕 |  | 九十七年一月一日起 | 使用於M1、N1類車輛以及總重量小於三‧五公噸之M2類車輛外側前座之新型式頭枕 |
|  |  | 九十九年一月一日起 | 使用於M1、N1類車輛以及總重量小於三‧五公噸之M2類車輛外側前座之各型式頭枕 |
| 五十之一、頭枕：自一０二年一月一日起實施 |  | 一０二年一月一日起 | 下列之頭枕，應符合本項規定：(1.1.1~1.1.2) |
|  |  | 一０四年一月一日起 | 使用於M1及總重量小於三‧五公噸之M2類車輛後座之各型式頭枕 |
| 五十一、門閂／鉸鏈 |  | 九十七年一月一日起 | 使用於M1及N1類車輛之新型式門閂/鉸鏈 |
|  |  | 九十九年一月一日起 | 使用於M1及N1類車輛之各型式門閂/鉸鏈 |
| 五十一之一、門閂與鉸鏈：自一０二年一月一日起實施 |  | 一０二年一月一日起 | 使用於M1及N1類車輛乘員進出門之新型式門閂與鉸鏈 |
|  |  | 一０四年一月一日起 | 使用於M1及N1類車輛乘員進出門之各型式門閂與鉸鏈已符合本基準項次「五十一」之規定者，其尾門應符合5.1.3、5.2.3、5.3.3.2.5、5.3.3.2.6、5.5.1.3之規定 |
| 五十二、非氣體放電式頭燈 |  | 九十七年一月一日起 | 使用於M及N類車輛之新型式非氣體放電式頭燈 |
|  |  | 九十九年一月一日起 | 使用於M及N類車輛之各型式非氣體放電式頭燈 |
|  |  | 九十八年一月一日起 | 使用於L1及L3類車輛之新型式非氣體放電式頭燈 |
|  |  | 一００年一月一日起 | 使用於L1及L3類車輛之各型式非氣體放電式頭燈 |
|  |  | 九十八年十二月二十五日起 | 修訂適用配光穩定性能試驗及塑膠透鏡性能試驗之頭燈種類 |
| 五十二之一、非氣體放電式頭燈：自一００年一月一日起實施 |  | 一００年一月一日起 | 使用於M、N、L1及L3類車輛之新型式非氣體放電式頭燈 |
|  |  | 一０二年一月一日起 | 使用於M、N、L1及L3類車輛之各型式非氣體放電式頭燈，已符合本基準項次「五十二」之規定且裝設額外光源及/或具備轉彎光型者，另應符合本項之裝設額外光源及/或具備轉彎光型之相關規定 |
|  |  | 九十八年十二月二十五日起 | 修訂適用配光穩定性能試驗、塑膠透鏡性能試驗及抗光源輻射之頭燈種類。 |
| 五十三、後霧燈 |  | 九十七年一月一日起 | 使用於M、N及O類車輛之新型式後霧燈 |
|  |  | 九十九年一月一日起 | 使用於M、N及O類車輛之各型式後霧燈，應符合本項規定，且應使用符合本基準中「燈泡」規定之燈泡 |
|  |  | 九十八年一月一日起 | 使用於L3類車輛之新型式後霧燈 |
|  |  | 一００年一月一日起 | 使用於L3類車輛之各型式後霧燈，應符合本項規定，且應使用符合本基準中「燈泡」規定之燈泡 |
| 五十四、火災防止規定 |  | 九十七年十二月三十一日起 | 軸距逾四公尺之大客車及軸距未逾四公尺且總重量逾四‧五噸之大客車 |
| 五十五、大客車車身結構強度 |  | 九十七年十二月三十一日起 | 下述車輛之車身結構強度，應符合本項規定 1.1 軸距逾四公尺之大客車 1.2 軸距未逾四公尺、總重量逾四‧五噸且乘員座立位總數逾二十二人（不包括駕駛員）之下列大客車： 1.2.1 僅設座位供載運乘客 1.2.2 設有座位供做載客用途，於走道或其他空間設有立位，而該其他空間不超過相當於二個雙人座椅空間 |
| 五十六、電磁相容性(修正草案) |  | 一００年一月一日起 | 新型式之L、M1及N1類車輛 |
|  |  | 一０二年一月一日起 | 各型式之L、M1及N1類車輛 |
| 五十六之一、電磁相容性：自一０四年一月一日起實施(草案) |  | 一０二年一月一日起 | 新型式之L類車輛；已符合「五十六、電磁相容性」者，另應符合電磁耐受規定。 |
|  |  | 一０四年一月一日起 | 各型式之L類車輛；已符合「五十六、電磁相容性」者，另應符合電磁耐受規定。 |
|  |  | 一０三年一月一日起 | 新型式之M、N及O類車輛；已符合「五十六、電磁相容性」者，另應符合電磁耐受規定。 |
|  |  | 一０五年一月一日起 | 各型式之M、N及O類電動車輛 |
|  |  | 一０五年一月一日起 | 各型式之M1、N1類非電動車輛 |
|  |  | 一０六年一月一日起 | 各型式之M2、M3、N2、N3類非電動車輛 |
|  |  | 一０七年一月一日起 | 各型式之O類非電動車輛 |
| 五十七、小型輕型機器腳踏車電子控制裝置 |  | 九十六年六月一日起 | 小型輕型機器腳踏車 |
| 五十八、小型輕型機器腳踏車之車架疲勞強度 |  | 九十六年七月一日起 | 小型輕型機器腳踏車 |
| 五十九、適路性前方照明系統 |  | 一００年一月一日起 | 使用於M及N類車輛之新型式適路性前方照明系統 |
|  |  | 一０二年一月一日起 | 使用於M及N類車輛之各型式適路性前方照明系統 |
| 六十、含視野輔助燈之照後鏡 |  | 公告日起 | 使用於M及N類車輛之照後鏡，如含後方視野輔助燈者，應符合本項規定。 |
| 六十一、機械式聯結裝置安裝規定 |  | 一００年一月一日起 | 除車輛安全檢測基準第十七項所指小型汽車及其附掛拖車以外之新型式N及O類車輛 |
|  |  | 一００年七月一日起 | 除車輛安全檢測基準第十七項所指小型汽車及其附掛拖車以外之各型式N及O類車輛 |
| 六十二、機械式聯結裝置 |  | 一００年一月一日起 | 除車輛安全檢測基準第十七項所指小型汽車及其附掛拖車以外之N及O類車輛所使用之機械式聯結裝置 |
| 六十三、低地板大客車規格規定 |  | 九九年八月十六日起 | 下述設有立位之新型式低地板大客車： 1.1 軸距逾四公尺之大客車。 1.2 軸距未逾四公尺且總重量逾四‧五噸之有設立位大客車。 |
| 六十四、電動汽車之電氣安全：自一０三年一月一日起實施(草案) |  | 一０三年一月一日起 | 使用於設計速度大於25公里/小時之新型式Ｍ及Ｎ類電動車輛 |
|  |  | 一０五年一月一日起 | 使用於設計速度大於25公里/小時之各型式Ｍ及Ｎ類電動車輛 |
| 六十五、電動機器腳踏車高溫擠壓電擊安全防護規範：自○年○月○日起實施(草案) |  | ○年○月○日起 | 新型式之L類電動機器腳踏車 |
|  |  | ○年○月○日起 | 各型式之L類電動機器腳踏車 |
| 六十六、燃油箱火災防止(草案) |  | ○年○月○日起 | 使用於Ｍ、Ｎ及O類車輛之新型式油箱 |
|  |  | ○年○月○日起 | 使用於Ｍ、Ｎ及O類車輛之各型式油箱 |
| 六十七、大客車設置之自行車置放架靜態強度(草案) |  | ○年○月○日起 | 設有自行車置放架之M2及M3類車輛 |

| 待公告車輛安全檢測基準增修案規劃表 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 項目 |  | 參考 | 草案年 | 適用車輛 |  | 實施日期綜合確認因子 |  |  | 實施年 |  | 預定公告年 |
| 編號 | 名稱 | ECE版本 |  | 基準範圍 | 增修對象 | ECE所訂新型式日期A後一年 | 討論定案後緩衝期兩年(定案年+2) | 依產品開發技術難度 | 新型式 | 既有型式 |  |
| 7-1 | 左右兩側防止捲入裝置與後方安全防護裝置(或保險槓) | --- | 99年 | N2、N3、O2、O3、O4 | N2、N3、O2、O3、O4類 | --- | 99+2 ->101(2012)年 | --- | 待討論 | 待討論 | 待討論 |
| 42-1 | 動態煞車 | 13-10-S5 13H-00-S5 | 98年 | M、N及O類車輛 | M、N及O類電動車輛 | --- | 98+2 ->100(2011)年 | --- | 100(2011)年7月 | 102(2013)年7月 配備電力再生煞車系統之電動車 | 99(2010)年 (交通部99/2/25討論) |
| 42-3 | 動態煞車 | 13H-00-S7 13H-00-S8 13H-00-R1/C1 13H-00-S7/C1 13-11 | 99年 | M、N、O、L1及L3類 | M1、N1，及 N2、N3、M2、M3其車輪軸未愈三軸者，及 O3、O4車輪軸未愈三軸者且配備氣壓式懸吊 | 2013/11/1(ESC系統選配) ->103(2014)年 （各車種有其不同對應之實施時間） | 99+2 ->101(2012)年 | 待討論 | 待討論 | 待討論 | 待討論 |
| 43-1 | 防鎖死煞車系統 | 78-03(-C1) | 97年 | M、N、O、L1及L3類 | L1及L3類 | 2009/06/18 ->99(2010)年 | 98+2 -->100(2011)年 | 102(2013)年 | 102(2013)年 | 106(2017)年 L1及L3類 | 99(2010)年 |
| 44-1 | 轉向控制系駕駛人碰撞保護 | 12-03-S3 | 98年 | M1、總重量小於1.5公噸之N1類 | M1及總重量小於1.5公噸之N1類電動車輛 | --- | 98+2 ->100(2011)年 | --- | 101(2012)年 | 103(2014)年 | 99(2010)年 (交通部99/2/25討論) |
| 45-1 | 側方碰撞乘員保護 | 95-ELSA-7-04 | 98年 | R點距地高小於700mm之M1及N1 | R點距地高小於700mm之M1及N1 | --- | 98+2 ->100(2011)年 | --- | 103(2014)年 | 103(2014)年 | 99(2010)年 (交通部99/2/25討論) |
| 46-1 | 前方碰撞乘員保護 | 94-ELSA-7-04 | 98年 | 總重量小於2.5公噸之M1類 | 總重量小於2.5公噸之M1類 | --- | 98+2 ->100(2011)年 | --- | 103(2014)年 | 103(2014)年 | 99(2010)年 (交通部99/2/25討論) |
| 56 | 電磁相容性 | 10-02-S2 | 98年 | L、M、N及O類車輛 | L、M1及N1類 | --- | 98+2 ->100(2011)年 | --- | 100(2011)年 | 102(2013)年 | 99(2010)年 (交通部99/2/25討論) |
| 56-1 | 電磁相容性 | 10-03 | 98年 | L、M、N及O類之車輛 | 新增EMS | 2010/7/11 ->100(2011)年 | 98+2 ->100(2011)年 | 104(2015)年 | L類:102(2013)年 MNO類:103(2014)年 | 1.L類:104(2015)年 2.MNO類電動車: 105(2016)年 3.M1、N1類非電動車: 105(2016)年 4.M2、M3、N2及N3類非電動車: 106(2017)年 5.O類非電動車: 107(2018年) 6.已符合基準項次「五十六」之規定者可免本項之EMI | 99(2010)年 (交通部99/2/25討論) |
| 64 | 電動汽車之電氣安全 | 100-ELSA-6-07 | 98年 | M及N類電動車 | M及N類電動車 | 2013/9 ->103(2014)年 | 98+2 ->100(2011)年 | --- | 103(2014)年 | 105(2016)年 | 99(2010)年 (交通部99/2/25討論) |
| 65 | 電動機器腳踏車高溫擠壓電擊安全防護規範 | --- | 98年 | L類電動機器腳踏車 | L類電動機器腳踏車 | --- | 98+2 ->100(2011)年 | --- | 99(2010)年9月 (暫定) | 101(2012)年9月 (暫定) | 99(2010)年 (交通部99/2/25討論) |
| 66 | 燃油箱火災防止 | 34-02-S2 | 99年 | M、N及O類 | M、N及O類車輛之燃油箱 | 2008/6/11 ->98(2009)年 | 99+2 ->101(2012)年 | 103(2014)年 | 待訂 (金屬:100年7月；塑膠:101年7月)或(金屬與塑膠:103年) | 待訂 (金屬:102年7月；塑膠:102年7月)或(金屬與塑膠:105年) | 99(2010)年 |
| 67 | 大客車設置之自行車置放架靜態強度 | --- | 99年 | M2及M3類 | M2及M3類 | --- | 99+2 ->101(2012)年 | --- | 待討論 | 待討論 | 待討論 |